JIFFYDOS

version 6.01

This document is a dump of JiffyDOS version 6.01. The parts concerning JiffyDOS have been commented by Magnus Nyman (magnus.q.nyman@telia.se), also known as Harlekin/FairLight. All new JiffyDOS routines are specifyed in the text. The comments to other parts of this document are partially rewritten from 'The Bible', Commodore Reference Manual. Some text has been added, to increment the knowledge of the original Commodore routines. Some errors have been corrected, and old routines has been removed. Also included are my own thoughts of what can be improved.

If you find any errors, feel free to contact me on the address above.

//Magnus

Document revision B. 1996-10-03

New Zeropage addresses in the JiffyDOS system.

$26       	; allflag/rsize    2

$27      	; comsav           2

$9b	; AKTFLG, flags if jiffyDOS funktionkeys is 	 	; aktivated. If 0, funktionkeys are enabled.

$9f	; CJLA, JiffyDOS default filenumber.

$A3       	; ldflg/qflag, Used in all disk access and LOAD 	  routines.

$a6	; TFLAG, temp store of JiffyDOS command number.

$b0/$b1	; KEYPTR, vector to table of JiffyDOS funktion keys.

$B0	; sprsav, Saved, then restored by LOAD routine (no 	  other usage).

$B1	; rassav           1

$B2	; regsav           1

$be	; DRVBYT, JiffyDOS default device number.

DISK ACCESS

The 64’er access the serial devices through the CIA at $DD00. The ”bits” have the following connection! The JiffyDOS performs it’s own timing and handshaking, allowing us to use both the data AND the clock lead to transfer data. This has two advantages. 1) We send/receive two bits at a time. 2) Because of the heavy timing that is done before the bits are sent, we don’t need any timing for the next bytes, and we can send 4*2 bits (ie. the entire byte) without any more timing!

adr $DD00	56576

bit 7	Serial Bus Data Input

6	Serial Bus Clock Pulse Input

5	Serial Bus Data Output

4	Serial Bus Clock Pulse Output

3	Serial Bus ATN Signal Output

Bits used

; E000

sta a56

jsr ebc0f

lda a61

cmp #$88

bcc ie00e

ie00b  jsr ebad4

ie00e  jsr ebccc

lda a07

clc

adc #$81

beq ie00b

sec

sbc #$01

pha

ldx #$05

ie01e  lda f69,x

ldy f61,x

sta f61,x

sty f69,x

dex

bpl ie01e

lda a56

sta a70

jsr eb853

jsr ebfb4

lda #$c4

ldy #$bf

jsr ie059

lda #$00

sta a6f

pla

jsr ebab9

rts

ie043  sta a71

sty a72

jsr ebbca

lda #$57

jsr eba28

jsr ie05d

lda #$57

ldy #$00

jmp eba28

ie059  sta a71

sty a72

ie05d  jsr ebbc7

lda (p71),y

sta a67

ldy a71

iny

tya

bne ie06c

inc a72

ie06c  sta a71

ldy a72

ie070  jsr eba28

lda a71

ldy a72

clc

adc #$05

bcc ie07d

iny

ie07d  sta a71

sty a72

jsr eb867

lda #$5c

ldy #$00

dec a67

bne ie070

rts

tya

and f44,x

ror $

brk

pla

plp

lda (p46),y

brk

jsr ebc2b

bmi ie0d3

bne ie0be

jsr efff3

stx a22

sty a23

ldy #$04

lda (p22),y

sta a62

iny

lda (p22),y

sta a64

ldy #$08

lda (p22),y

sta a63

iny

lda (p22),y

sta a65

jmp ie0e3

ie0be  lda #$8b

ldy #$00

jsr ebba2

lda #$8d

ldy #$e0

jsr eba28

lda #$92

ldy #$e0

jsr eb867

ie0d3  ldx a65

lda a62

sta a65

stx a62

ldx a63

lda a64

sta a63

stx a64

ie0e3  lda #$00

sta a66

lda a61

sta a70

lda #$80

sta a61

jsr eb8d7

ldx #$8b

ldy #$00

ie0f6  jmp ebbd4

E0F9 BIOERR: HANDLE I/O ERROR IN BASIC

This routine is called whenever BASIC wishes to call one of the KERNAL I/O routines. It is also used to handle I/O errors in BASIC.

.e0f9  cmp #$f0	; test error

bne $e104

sty $38	; MEMSIZ, highest address in BASIC

stx $37

jmp $a663	; do CLR without aborting I/O

.e104  tax	; put error flag i (X)

bne $e109	; if error code $00, then set error code $1e

ldx #$1e

.e109  jmp $a437	; do error

E10C BCHOUT: OUTPUT CHARACTER

This routine uses the KERNAL rutine CHROUT to output the character in (A) to an available output channel. A test is made for a possible I/O error.

.e10c  jsr $ffd2	; output character in (A)

bcs $e0f9	; if carry set, handle I/O error

rts	; else return

E112 BCHIN: INPUT CHARACTER

This routine uses the KERNAL routine CHRIN to input a character to (A) from an available input channel. A test is made for a possible I/O error.

.e112  jsr $ffcf	; input character from CHRIN

bcs $e0f9	; if carry set, handle I/O error

rts	; else return

E118 BCKOUT:SET UP FOR OUTPUT

This routine uses the KERNAL routine CHKOUT to open an output channel, and tests for possible I/O error. On entry (X) must hold the the logical file number as used in OPEN.

.e118  jsr $e4ad	; open output channel via CHKOUT

bcs $e0f9	; if carry set, handle I/O error

rts	; else return

E11E BCKIN: SET UP FOR INPUT

This routine uses the KERNAL routine CHKIN to open an input channel. A test as made for possible I/O error.

.e11e  jsr $ffc6	; open input channel via CHKIN

bcs $e0f9	; if carry set, handle I/O error

rts	; else return

E124 BGETIN: GET ONT CHARACTER

This routine uses the KERNAL routine GETIN to get a character from the keyboard buffer into (A). A test is made for possible I/O error.

.e124  jsr $ffe4	; GETIN, get character from keyboard buffer

bcs $e0f9	; if carry set, handle I/O error

rts	; else return

E12A SYS: PERFORM SYS

This routine enables machine language routines to be executed from BASIC. The routine evaluates the address and confirms that it is a numeric number. The return address is set up, and the user routine is executed.

.e12a  jsr $ad8a	; evaluate text & confirm numeric

jsr $b7f7	; convert fac#1 to integer in LINNUM

lda #$e1	; set return address on stack to $ea46

pha

lda #$46

pha

lda $030f	; SPREG, user flag register

pha

lda $030c	; SAREG, user (A) register

ldx $030d	; SXREG, user (X) register

ldy $030e	; SYREG, user (Y) register

plp

jmp ($14)	; execute user routine, exit with rts

.e146  php

sta $030c	; store in SAREG, user (A) register

stx $030d	; store in SXREG, user (X) register

sty $030e	; store in SYREG, user (Y) register

pla

sta $030f	; store in SPREG, user flag register

rts	; back

E156 SAVET: PERFORM SAVE

This routine is sets parameters for save, and calls the save routine. The start and end addresses are obtained from TXTTAB and VARTAB. Finally, a test is made if any errors ocured.

jsr $e1d4	; get SAVE paramerters from text

ldx $2d	; VARTAB, start of variables

ldy $2e

lda #$2b	; <TXTTAB, start of BASIC text

jsr $ffd8	; execute SAVE

bcs $e0f9	; if carry is set, handle I/O errors

rts

E165 VERFYT: PERFORM LOAD/SAVE

This routine is essentially the same for both LOAD and VERIFY. The entry point determins which is performed, by setting VERCK accordingly. The LOAD/VERIFY parameters, filename, device etc. are obtained from text before the KERNAL routine LOAD is called. A test is made for I/O errors. At this point, the two functios are distiguished. VERIFY reads the the status word and prints the message OK or ?VERIFY error depending on the result of the test. LOAD reads the I/O status word for a possible ?LOAD error, then updates the pointers to text and variables, exiting via CLR.

.e165  lda #$01	; flag verify

bit $00a9	; mask, will execute lda #$01 if address $e168

sta $0a	; store in VRECK, LOAD/VERIFY flag

jsr $e1d4	; get LOAD/VERIFY parameters from text

lda $0a	; get VRECK

ldx $2b	; TXTTAB, start of BASIC

ldy $2c

jsr $ffd5	; execute LOAD, KERNAL routine

bcs $e1d1	; if carry set, handle error

lda $0a	; test VRECK for LOAD or VERIFY

beq $e195	; do LOAD

ldx #$1c	; set error $1c, VERIFY error

jsr $ffb7	; do READST, get status I/O word

and #$10	; %00010000, test for mismatch

bne $e19e	; data mismatch, do error

lda $7a	; <TXTPTR

cmp #$02

beq $e194

lda #$64	; set address to text OK

ldy #$a3	; at $a364

jmp $ab1e	; output string in (A/Y)

.e194  rts

.e195  jsr $ffb7	; do READST, get status I/O for LOAD

and #$bf	; %10111111, test all but EOI

beq $e1a1	; nope, no errors

ldx #$1d	; set error $1d, LOAD error

.e19e  jmp $a437	; do error

.e1a1  lda $7b	; >TXTPTR

cmp #$02

bne $e1b5

stx $2d	; set VARTAB, start of variables

sty $2e

lda #$76	; set address to text READY

ldy #$a3	; at $a376

jsr $ab1e	; output string in (A/Y)

jmp $a52a	; do CLR and restart BASIC

.e1b5  jsr $a68e	; reset TXTPTR

jsr $a533	; rechain BASIC lines

jmp $a677	; do RESTORE and reset OLDTXT

E1BE OPENT: PERFORM OPEN

This routine extracts paramerters from text and performs the OPEN routine in KERNAL. A test is made for I/O errors.

.e1be  jsr $e219	; get parameters from text

jsr $ffc0	; execute OPEN

bcs $e1d1	; if carry set, handle error

rts

E1C7 CLOSET: PERFORM CLOSE

The parameters for CLOSE are obtained from text, and the logical filenumber placed in (A), The KERNAL routine CLOSE is performed, and a test is made for I/O errors.

.e1c7  jsr $e219	; get parameters from text

lda $49	; logical file number

jsr $ffc3	; perform CLOSE

bcc $e194	; if carry set, handle error, else return

.e1d1  jmp $e0f9	; jump to error routine

E1D4 SLPARA: GET PARAMETERS FOR LOAD/SAVE

This routine gets the filename, devicenumber and secondary address for LOAD/VERIFY and SAVE operations. The KERNAL routines SETNAM and SETLFS are used to do this. Default parameters are set up, and a new JiffyDOS routine is called at $e1dd. It jumps to $f73a where the original SETLFS is performed, but also makes a test to find the first serial device number, and pokes it into FA. Then tests are made if any of the parameters were given. If so, these are set up as wanted.

.e1d4  lda #$00	; clear length of filename

jsr $ffbd	; SETNAM

ldx #$01	; default FA, device number is #01

ldy #$00	; default SA, secondary address is #00

.e1dd  jsr $f73a	; SETLFS, and device number in new JiffyDOS routine

jsr $e206	; test if "end of line", if so end here

jsr $e257	; set up given filename and perform SETNAM

jsr $e206	; test if "end of line", if so end here

jsr $e200	; check for comma, and input one byte, FA, to (X)

ldy #$00

stx $49

jsr $ffba	; perform new SETLFS with device number

jsr $e206	; test if "end of line", if so end here

jsr $e200	; check for comma, and input one byte, SA, to (X)

txa	; transfer (X) to (Y)

tay

ldx $49	; get FA

jmp $ffba	; perform SETLFS with both device number and secondary 	  address. Then exit

E200 COMBYT: GET NEXT ONE-BYTE PARAMETER

This routine checks if the next character of text is a comma, and then inputs the parameter following into (X).

.e200  jsr $e20e	; check for comma

jmp $b79e	; input one byte parameter to (X)

E206 DEFLT: CHECK DEFAULT PARAMETERS

This routine tests CHRGOT to see if a optional parameter was included in the text. If it was, a normal exit is performed via RTS. If not, the return address on the stack is discarded, and the routine exits both this and the calling routine.

.e206  jsr $79	; get CHRGOT

bne $e20d	; if last character is a character, do normal exit

pla	; else, remove return address

pla	; to exit this AND the calling routine.

.e20d  rts	; exit

E20E CMMERR: CHECK FOR COMMA

This routine confirms that the next character in the text is a comma. It also test that the comma is not immediately followed by a terminator. If so, exit and do SYNTAX error.

.e20e  jsr $aefd	; confirm comma

.e211  jsr $79	; get CHRGOT

bne $e20d	; else than null

jmp $af08	; execute SYNTAX error

E219 OCPARA: GET PARAMETERS FOR OPEN/CLOSE

This routine gets the logical file number, device number, secondary address and filename for OPEN/CLOSE. Initially the default filename is set to null, and the device number to #1. The logical filenumber is compulsory, and is obtained from text and placed in <FORPNT. The other parameters are optinal and are obtained if present. The device number is stored in >FORPNT. The parameters are set via the KERNAL routines SETNAM and SETLFS.

.e219  lda #$00	; default filename is null

jsr $ffbd	; SETNAM

jsr $e211	; confirm TXTPNT is no terminator, if so - error

jsr $b79e	; input one byte character to (X)

stx $49	; store logical filenumber in <FORPNT

txa	; set default parameters to

ldx #$01	; device = #1

ldy #$00	; secondary address = #0

jsr $ffba	; SETLFS

jsr $e206	; test if "end of line", if so end here

jsr $e200	; check for comma, and input FA, device number

stx $4a	; store in >FORPNT

ldy #$00	; secondary address = #0

lda $49	; logical file number from temp store

cpx #$03	; test if serial devce

bcc $e23f	; nope

dey	; if serial, set secondary address to $ff

.e23f  jsr $ffba	; SETLFS

jsr $e206	; test if "end of line", if so end here

jsr $e200	; check for comma, and input SA, secondary address

txa

tay	; SA to (Y)

ldx $4a	; FA

lda $49	; LA

jsr $ffba	; SETLFS

jsr $e206	; test if "end of line", if so end here

jsr $e20e	; check for comma only

.e257  jsr $ad9e	; evaluate expression in text

jsr $b6a3	; do string housekeeping

ldx $22	; pointers to given filename

ldy $23

jmp $ffbd	; SETNAM and exit

E264 COS: PERFORM COS

This routine manipulates the input COS to be calcuated with SIN. COS(X) = SIN(X+pi/2), where  X is in radians. We use it as Fac#1=SIN(fac#1+pi/2), ie pi/2 is added to fac#1 and the following SIN is performed.

lda #$e0	; set address to pi/2

ldy #$e2	; at $e2e0

jsr $b867	; add fltp at (A/Y) to fac#1

E26B SIN: PERFORM SIN

ie26b  jsr ebc0c

lda #$e5

ldy #$e2

ldx a6e

jsr ebb07

jsr ebc0c

jsr ebccc

lda #$00

sta a6f

jsr eb853

lda #$ea

ldy #$e2

jsr eb850

lda a66

pha

bpl ie29d

jsr eb849

lda a66

bmi ie2a0

lda a12

eor #$ff

sta a12

ie29d  jsr ebfb4

ie2a0  lda #$ea

ldy #$e2

jsr eb867

pla

bpl ie2ad

jsr ebfb4

ie2ad  lda #$ef

ldy #$e2

jmp ie043

E2B4 TAN: PERFORM TAN

.e2b4  jsr ebbca

lda #$00

sta a12

jsr ie26b

ldx #$4e

ldy #$00

jsr ie0f6

lda #$57

ldy #$00

jsr ebba2

lda #$00

sta a66

lda a12

jsr ie2dc

lda #$4e

ldy #$00

jmp ebb0f

ie2dc  pha

jmp ie29d

E2E0 PI2: TABLE OF TRIGONOMITRY CONSTANTS

The following constants are held in 5 byte flpt for trigonomitry evaluation.

.e2e0  81 49 0f da a2	; 1.570796327 (pi/2)

.e2e5  83 49 0f da a2	; 6.28318531  (pi*2)

.e2ea  7f 00 00 00 00	; 0.25

.e2ef  05	; 5 (one byte counter for SIN series)

.e2f0  84 e6 1a 2d 1b	; -14.3813907 (SIN constant 1)

.e2f5  86 28 07 fb f8	; 42.0077971  (SIN constant 2)

.e2fa  87 99 68 89 01	; -76.7041703 (SIN constant 3)

.e2ff  87 23 35 df e1	; 81.6052237  (SIN constant 4)

.e304  86 a5 5d e7 28	; -41.3417021 (SIN constant 5)

.e309  83 49 0f ds a2	; 6.28318531  (SIN constant 6, pi*2)

E30E ATN: PERFORM ATN

.e30e  lda $66

pha

bpl ie316

jsr ebfb4

ie316  lda a61

pha

cmp #$81

bcc ie324

lda #$bc

ldy #$b9

jsr ebb0f

ie324  lda #$3e

ldy #$e3

jsr ie043

pla

cmp #$81

bcc ie337

lda #$e0

ldy #$e2

jsr eb850

ie337  pla

bpl ie33d

jmp ebfb4

ie33d  rts

E33E ATNCON: TABLE OF ATN CONSTANTS

The table holds a 1 byte counter and the folloeing 5 byte flpt constants.

.e33e  0b	; 13 (one byte counter for ATN series)

.e33f  76 b3 83 bd d3	; -0.000684793912 (ATN constant 1)

.e344  79 1e f4 a6 f5	; 0.00485094216   (ATN constant 2)

.e349  7b 83 fc b0 10 	; -0.161117018    (ATN constant 3)

.e34e  7c 0c 1f 67 ca	; 0.034209638     (ATN constant 5)

.e353  7c de 53 cb c1	; -0.0542791328   (ATN constant 6)

.e358  7d 14 64 70 4c	; 0.0724571965    (ATN constant 7)

.e35d  7d b7 ea 51 7a	; -0.0898023954   (ATN constant 8)

.e362  7d 63 30 88 7e	; 0.110932413     (ATN constant 9)

.e367  7e 92 44 99 3a	; -0.14283908     (ATN constant 10)

.e36c  7e 4c cc 91 c7	; 0.19999912      (ATN constant 11)

.e371  7f aa aa aa 13	; -0.333333316    (ATN constant 12)

.e376  81 00 00 00 00	; 1               (ATN constant 13)

E37B BASSFT: BASIC WARM START

This is the BASIC warm start routine that is vectored at the very start of the BASIC ROM. The routine is called by the 6510 BRK instruction, or STOP/RESTORE being pressed. It outputs the READY prompt via the IERROR vector at $0300. The original IERROR vector points to $e38b, but JiffyDOS uses the error routine as an input to check new commands. If the error code, in (X) is larger than $80, then only the READY text will be displayed.

.e37b  jsr $ffcc	; CLRCHN, close all I/O channels

lda #$00

sta $13	; input prompt flag

jsr $a67a	; do CLR

cli	; enable IRQ

.e386  ldx #$80	; error code #$80

jmp ($0300)	; perform error, JiffyDOS at $f763

.e38b  txa	; error number

bmi $e391	; larger than $80

jmp $a43a	; nope, print error

.e391  jmp $a474	; print READY

E394 INIT: BASIC COLD START

This is the BASIC cold start routine that is vectored at the very start of the BASIC ROM. BASIC vectors and variables are set up, and power-up message is output, and BASIC is restarted.

.e394  jsr $e4b7	; Init JiffyDOS commands & funktionkeys

jsr $e3bf	; Initialize BASIC

jsr $e422	; output power-up message

ldx #$fb	; reset stack

txs

bne $e386	; output READY, and restart BASIC

E3A2 INITAT: CHRGET FOR ZEROPAGE

This is the CHRGET routine which is transferred to RAM starting at $0073 on power-up or reset.

.e3a2  inc $7a	; .0073  inc $7a	; increment <TXTPTR

bne $e3a8	;        bne $0079	; skip high byte

inc $7b	;        inc $7b	; increment >TXTPTR

.e3a8  lda $ea60	; .0079  lda $ea60	; CHRGOT entry, read TXTPTR

cmp #$3a	;        cmp #$3a	; colon (terminator), sets (Z)

bcs $e3b9	;        bcs $008a

cmp #$20	;        cmp #$20	; space, get next character

beq $e3a2	;        bne $0073

sec	;        sec

sbc #$30	;        sbc #$30	; zero

sec	;        sec

sbc #$d0	;        sbc #$d0

.e3b9  rts	; .008a  rts

E3BA RNDSED: RANDOM SEED FOR ZEROPAGE

This is the initial value of the seed for the random number function. It is copied into RAM from $008b-$008f. Its fltp value is 0.811635157.

.e3ba  80 4f c7 52 58

E3BF INITCZ: INITIALISE BASIC RAM

This routine sets the USR jump instruction to point to ?ILLIGAL QUANTITY error, sets ADRAY1 and ADRAY2, copies CHRGET and RNDSED to zeropage, sets up the start and end locations for BASIC text and sets the first text byte to zero.

.e3bf  lda #$4c	; opcode for JMP

sta $54	; store in JMPER

sta $0310	; USRPOK, set USR JMP instruction

lda #$48

ldy #$b2	; vector to $b248, ?ILLIGAL QUANTITY

sta $0311

sty $0312	; store in USRADD

lda #$91

ldy #$b3	; vector to $b391

sta $05

sty $06	; store in ADRAY2

lda #$aa

ldy #$b1	; vector to $b1aa

sta $03

sty $04	; store in ADRAY1

ldx #$1c	; copy the CHRGET routine and RNDSED to RAM

.e3e2  lda $e3a2,x	; source address

sta $73,x	; destination address

dex	; next byte

bpl $e3e2	; till ready

lda #$03

sta $53	; store #3 in FOUR6, garbage collection

lda #$00

sta $68	; init BITS, fac#1 overflow

sta $13	; init input prompt flag

sta $18	; init LASTPT

ldx #$01

stx $01fd

stx $01fc

ldx #$19

stx $16	; TEMPPT, pointer to descriptor stack

sec	; set carry to indicate read mode

jsr $ff9c	; read MEMBOT

stx $2b	; set TXTTAB, bottom of RAM

sty $2c

sec	; set carry to indicate read mode

jsr $ff99	; read MEMTOP

stx $37	; set MEMSIZ, top of RAM

sty $38

stx $33	; set FRETOP = MEMTOP

sty $34

ldy #$00

tya

sta ($2b),y	; store zero at start of BASIC

inc $2b	; increment TXTTAB to next memory position

bne $e421	; skip msb

inc $2c

.e421  rts	; return

E422 INITMS: OUTPUT POWER-UP MESSAGE

This routine outputs the startup message. It then calcuates the number of BASIC bytes free by subatracting the TXTTAB from MEMSIZ, and outputs this number. The routine exits via NEW.

.e422  lda $2b	; read TXTTAB, start of BASIC

ldy $2c

jsr $a408	; check for memory overlap

lda #$73	; $e473, startup message

ldy #$e4

jsr $ab1e	; output (A/Y)

lda $37	; MEMSIZ, highest address in BASIC

sec	; prepare for substract

sbc $2b	; substract TXTTAB

tax	; move to (X)

lda $38	; and highbyte

sbc $2c

jsr $bdcd	; output number in (A/X)

lda #$60	; $e460

ldy #$e4	; pointer to 'BASIC BYTES FREE'

jsr $ab1e	; output (A/Y)

jmp $a644	; perform NEW

E447 JIFFYDOS VECTORS

This table contains jump vectors that are transfered to $0300-$030b. Some vectors are standard Commodore, but some are modifyed for JiffyDOS.

.e447  63 f7	; IERROR VEC, print basic error message ($f763)

; Original IERROR VEC points to $e38b

.e449  83 4a	; IMAIN VECTOR, basic warm start ($a483)

.e44b  64 ea	; ICRNCH VECTOR, tokenise basic text ($ea64)

; Original ICRNCH VECTOR points to $a57c

.e44d  a1 a7	; IQPLOP VECTOR, list basic text ($a7a1)

.e44f  e4 a7	; IGONE VEXTOR, basic character dispatch ($a7ea)

.e451  86 ea	; IEVAL VECTOR, evaluate basic token ($ae86)

E453 INIT JIFFYDOS COMMANDS

This routine transfers the vectors $0300-$030b to set up the JiffyDOS commands.

.e453  ldx #$0b	; 6 vectors to be copied

.e455  lda $e447,x

sta $0300,x

dex	; next byte

bpl $e455	; ready

rts	; return

E45F WORDS: POWER UP MESSAGE

This is the power up message displayed on the screen when the 'Commie' is switched on or reset. The strings are seperated by a zero byte.

jiffydos v6.01 (c)1989 cmd

c-64 basic v2  xxxxx basic bytes free

.e45f  00 20 42 41 53 49 43 20  ;   basic

.e467  42 59 54 45 53 20 46 52  ; bytes fr

.e46f  45 45 0d 00 93 0d 20 20  ; ee

.e477  20 20 20 20 20 4a 49 46  ;      jif

.e47f  46 59 44 4f 53 20 56 36  ; fydos v6

.e487  2e 30 31 20 28 43 29 31  ; .01 (c)1

.e48f  39 38 39 20 43 4d 44 20  ; 989 cmd

.e497  20 0d 0d 20 43 2d 36 34  ;     c-64

.e49f  20 42 41 53 49 43 20 56  ;  basic v

.e4a7  32 20 20 20 00 81        ; 2

E4AD PATCH FOR BASIC CHKOUT CALL

This is a short patch added for the KERNAL ROM to preserv (A) when there was no error returned from BASIC calling the CHKOUT routine. This corrects a bug in the early versions of PRINT# and CMD.

.e4ad  pha	; temp store (A)

jsr $ffc9	; CHKOUT

tax

pla	; retrieve (A)

bcc $e4b6

txa

.e4b6  rts

E4B7 INIT JIFFYDOS COMMANDS AND FUNKTIONKEYS

This routine initialises the JiffyDOS commands by jumping to $e453 where the $0300-vectors are set up. Then it sets up the vectors at $b0 to point to the funktionkey table at $f672. The entry at $e4c2 disables the funktionkeys after a @f command.

.e4b7  jsr $e453	; init JiffyDOS command vectors

lda #$72	; Set up JiffyDOS function key vector

sta $b0	; to $f672

lda #$f6

sta $b1

.e4c2  inx	; (X)=0

stx $9b	; AKTFLT, aktivate/deaktivate funktion keys

rts

.e4c6  lda #$6f	; #$6f=command channel

jsr $f0e4	; prepare for input

jsr $ffcf	; input byte from command channel

cmp #$35	; equal to #$35 (#)

rts

tax	; free byte

tax	; free byte

E4D3 RS232 PATCH

This patch has been added to the RS232 input routine in KERNAL v.3. It initialises the RS232 parity byte, RIPRTY, on reception of a start bit.

.e4d3  sta $a9	; RINONE, check for start bit

lda #$01

sta $ab	; RIPRTY, RS232 input parity

rts

E4DA RESET CHARACTER COLOUR

This routine is a patch in KERNAL version 3 to fix a bug with the colour code. The routine is called by 'clear a screen line', and sets the character colour to COLOR.

.e4da  lda $0286	; get COLOR

sta ($f3),y	; and store in current screen position

rts

E4E0 PAUSE AFTER FINDING TAPE FILE?????????????

This routine would continue tape loading without pressing C= when a file was found. This could probably be removed, since JiffyDOS not uses tape junk.

.e4e0  adc #$02

.e4e2  ldy $91

iny

bne $e4eb

cmp $a1

bne $e4e2

.e4eb  rts

E4EC RS232 TIMING TABLE - PAL

Timingtable for RS232 NMI for use with PAL machines. This table contains the prescaler values for setting up the RS232 baudrates. The table containe 10 entries which corresponds to one of the fixed RS232 rates, starting with lowest (50 baud) and finishing with the highest (2400 baud). Since the clock frequency is different between NTSC and PAL systems, there is another table for NTSC machines at $fec2.

.e4ec  19 29	; 50 baud

.e4ee  44 19	; 75 baud

.e4f0  1a 11	; 110 baud

.e4f2  e8 0d	; 134.5 baud

.e4f4  70 0c	; 150 baud

.e4f6  06 06	; 300 baud

.e4f8  d1 02	; 600 baud

.e4fa  37 01	; 1200 baud

.e4fc  ae 00	; (1800) 2400 baud

.e4fe  69 00	; 2400 baud

E500 IOBASE: GET I/O ADDRESS

The KERNAL routine IOBASE ($fff3) jumps to this routine. It returns the base address $dc00 in (X/Y)

.e500  ldx #$00	; set (X/Y) to $dc00

ldy #$dc

rts

E505 SCREEN: GET SCREEN SIZE

The KERNAL routine SCREEN ($ffed) jumps to this routine. It returns the screen size; columns in (X) and rows in (Y).

.e505  ldx #$28	; 40 columns

ldy #$19	; 25 rows

rts

E50A PLOT: PUT/GET ROW AND COLUMN

The KERNAL routine PLOT ($fff0) jumps to this routine. The option taken depends on the state of carry on entry. If it is set, the column is placed in (Y) and the row placed in (X). If carry is clear, the cursor position is read from (X/Y) and the screen pointers are set.

.e50a  bcs $e513	; if carry set, jump

stx $d6	; store TBLX, current row

sty $d3	; store PNTR, current column

jsr $e56c	; set screen pointers

.e513  ldx $d6	; read TBLX

ldy $d3	; read PNTR

rts

E518 CINT1: INITIALISE I/O

This routine is part of the KERNAL CINT init routine. I/O default values are set, <shift+cbm> keys are disabled, and cursor is switched off. The vector to the keyboard table is set up, and the length of the keyboardbuffer is set to 10 characters. The cursor color is set to lightblue, and the key-repeat parameters are set up.

.e518  jsr $e5a0	; set I/O defaults

lda #$00

sta $0291	; disable <SHIFT + CBM> by writing zero into MODE

sta $cf	; the cursor blink flag, set BLNON on

lda #$48

sta $028f

lda #$eb	; set the KEYLOG vector to point at $eb48

sta $0290

lda #$0a	; set max number of character is keyboard buffer to 10

sta $0289	; XMAX

sta $028c	; How many 1/60 of a second to wait before key is 	  repeated. Used togeather with $028b

lda #$0e	; set character colour to light blue

sta $0286	; COLOR

lda #$04	; How many $028c before a new entry is

sta $028b	; put in the keyboard buffer, KOUNT

lda #$0c

sta $cd	; store in BLCNT, cursor toggle timer

sta $cc	; store in BLNSW, cursor enable

E544 CLEAR SCREEN

This routine sets up the screen line link table ($d9 - $f2), LDTB1, which is used to point out the address to the screen. The later part of the routine performs the screen clear, line by line, starting at the bottom line. It continues to the next routine which is used to home the cursor.

.e544  lda $0288	; get HIBASE, top of screen memory

ora #$80	; fool around

tay

lda #$00

tax

.e54d  sty $d9,x	; store in screen line link table, LDTB1

clc

adc #$28	; add #40 to next line

bcc $e555

iny	; inc page number

.e555  inx	; next

cpx #$1a	; till all 26?? is done

bne $e54d

lda #$ff

sta $d9,x	; last pointer is $ff

ldx #$18	; start clear screen with line $18 (bottom line)

.e560  jsr $e9ff	; erase line (X)

dex	; next

bpl $e560	; till screen is empty

E566 HOME CURSOR

This routine puts the cursor in the top left corner by writing its column and line to zero.

.e566  ldy #$00

sty $d3	; write to PNTR, cursor column

sty $d6	; write to TBLX, line number

E56C SET SCREEN POINTRES

This routine positions the cursor on the screen and sets up the screen pointers. On entry, TBLX must hold the line number, and PNTR the column number of the cursor position. A major bug has been removed from the original commodore KERNAL. It sometimes caused the computer to crash, when deleting characters from the bottom line.

.e56c  ldx $d6	; read TBLX

lda $d3	; read PNTR

.e570  ldy $d9,x	; read value from screen line link table, LDTB1

bmi $e57c	; heavy calcuations??? jump when ready

clc

adc #$28

sta $d3	; PNTR

dex

bpl $e570

.e57c  jsr $e9f0	; set start of line (X)

lda #$27

inx

.e582  ldy $d9,x	; LDTB1

bmi $e58c

clc

adc #$28

inx

bpl $e582

.e58c  sta $d5	; store in LMNX, physical screen line length

jmp $ea24	; sync color pointer

.e591  cpx $c9	; read LXSP, chech cursor at start of input

beq $e598

jmp $e6ed	; retreat cursor

.e598  rts

nop	; A free byte!!! (own serial number haha)

E59A SET I/O DEFAULTS

The default output device is set to 3 (screen), and the default input device is set to 0 (keyboard). The VIC chip registers are set from the video chip setup table. The cursor is then set to the home position.

.e59a  jsr $e5a0	; set I/O defaults

jmp $e566	; home cursor and exit routine

.e5a0  lda #$03

sta $9a	; DFLTO, default output device - screen

lda #$00

sta $99	; DFLTN, default input device - keyboard

ldx #$2f

.e5aa  lda $ecb8,x	; VIC chip setup table

sta $cfff,x	; VIC chip I/O registers

dex	; next

bne $e5aa	; till ready

rts

E5B4 LP2: GET CHARACTER FROM KEYBOARD BUFFER

It is assumed that there is at leaset one character in the keyboard buffer. This character is obtained and the rest of the queue is moved up one by one to overwrite it. On exit, the character is in (A).

.e5b4  ldy $0277	; read KEYD, first character in keyboard buffer queue

ldx #$00

.e5b9  lda $0278,x	; overwrite with next in queue

sta $0277,x

inx

cpx $c6	; compare with NDX, number of characters in queue

bne $e5b9	; till all characters are moved

dec $c6 	; decrement NDX

tya	; transfer read character to (A)

cli	; enable interrupt

clc

rts

E5CA INPUT FROM KEYBOARD

This routine uses the previous routine to get characters from the keyboard buffer. Each character is output to the screen, unless it is <shift/RUN>. If so, the contents of the keyboard buffer is replaced with LOAD <CR> RUN <CR>. The routine ends when a carriage routine is encountered. The JSR at $e5e7 is o patch in JiffyDOS to test if the F-keys or other valid JiffyDOS keys are pressed. If not, this routine continues as normal.

.e5ca  jsr $e716	; output to screen

.e5cd  lda $c6	; read NDX, number of characters in keyboard queue

sta $cc	; BLNSW, cursor blink enable

sta $0292	; AUTODN, auto scroll down flag

beq $e5cd	; loop till key is pressed

sei	; disable interrupt

lda $cf	; BLNON, last cursor blink (on/off)

beq $e5e7

lda $ce	; GDBLN, character under cursor

ldx $0287	; GDCOL, background color under cursor

ldy #$00

sty $cf	; clear BLNON

jsr $ea13	; print to screen

.e5e7  jsr $f9e5	; Get character from keyboard buffer. JiffyDOS fixx

cmp #$83	; test if <shift/RUN> is pressed

bne $e5fe	; nope

ldx #$09	; transfer 'LOAD <CR> RUN <CR>' to keyboard buffer

sei

stx $c6	; store #9 in NDX, characters in buffer

.e5f3  lda $ece6,x	; 'LOAD <CR> RUN <CR>' message in ROM

sta $0276,x	; store in keyboard buffer

dex

bne $e5f3	; all nine characters

beq $e5cd	; allways jump

.e5fe  cmp #$0d	; carriage return pressed?

bne $e5ca	; nope, go to start

ldy $d5	; get LNMX, screen line length

sty $d0	; CRSV, flag input/get from keyboard

.e606  lda ($d1),y	; PNT, screen address

cmp #$20	; space?

bne $e60f	; nope

dey

bne $e606	; next

.e60f  iny

sty $c8	; store in INDX, end of logical line for input

ldy #$00

sty $0292	; AUTODN

sty $d3	; PNTR, cursor column

sty $d4	; QTSW, reset quoute mode

lda $c9	; LXSP, corsor X/Y position

bmi $e63a

ldx $d6	; TBLX, cursor line number

jsr $e591	; retreat cursor

cpx $c9	; LXSP

bne $e63a

lda $ca

sta $d3	; PNTR

cmp $c8	; INDX

bcc $e63a

bcs $e65d

E632 INPUT FROM SCREEN OR KEYBOARD

This routine is used by INPUT to input data from devices not on the serial bus, ie. from screen or keyboard. On entry (X) and (Y) registers are preserved. A test is made to determine which device the input is to be from. If it is the screen, then quotes and <RVS> are tested for and the character is echoed on the screen. Keyboard inputs make use of the previous routine.

.e632  tya	; preserve (X) and (Y) registers

pha

txa

pha

lda $d0	; CRSW, INPUT/GET from keyboard or screen

beq $e5cd	; input from keyboard

.e63a  ldy $d3	; PNTR, cursor column

lda ($d1),y	; read from current screen address

sta $d7	; temp store

and #$3f

asl $d7

bit $d7

bpl $e64a

ora #$80

.e64a  bcc $e650

ldx $d4	; QTSW, editor in quotes mode

bne $e654	; yepp

.e650  bvs $e654

ora #$40

.e654  inc $d3	; PNTR

jsr $e684	; do quotes test

cpy $c8	; INDX, end of logical line for input

bne $e674

.e65d  lda #$00

sta $d0	; CRSW

lda #$0d

ldx $99	; DFLTN, default input device

cpx #$03	; screen

beq $e66f	; yes

ldx $9a	; DFLTO, default output device

cpx #$03	; screen

beq $e672	; yes

.e66f  jsr $e716	; output to screen

.e672  lda #$0d

.e674  sta $d7

pla

tax	; restore (X) and (Y) registers

pla

tay

lda $d7

cmp #$de

bne $e682

lda #$ff

.e682  clc

rts

E684 QUOTES TSET

On entry, (A) holds the character to be tested. If (A) holds ASCII quotes, then the quotes flag is toggled.

.e684  cmp #$22	; ASCII quotes (")

bne $e690	; nope, return

lda $d4	; QTSW, quotes mode flag

eor #$01	; toggle on/off

sta $d4	; store

lda #$22	; restore (A) to #$22

.e690  rts

E691 SET UP SCREEN PRINT

The RVS flag is tested to see if reversed characters are to be printed. If insert mode is on, the insert counter is decremented by one. When in insert mode, all characters will be displayd, ie. DEL RVS etc. The character colour is placed in (X) and the character is printed to the scrren and the cursor advanced.

.e691  ora #$40

.e693  ldx $c7	; test RVS, flag for reversed characters

beq $e699	; nope

.e697  ora #$80	; set bit 7 to reverse character

.e699  ldx $d8	; test INSRT, flag for insert mode

beq $e69f	; nope

dec $d8	; decrement number of characters left to insert

.e69f  ldx $0286	; get COLOR, current character colour code

jsr $ea13	; print to screen

jsr $e6b6	; advance cursor

.e6a8  pla

tay

lda $d8	; INSRT

beq $e6b0

lsr $d4

.e6b0  pla

tax

pla

clc

cli

rts

E6B6 ADVANCE CURSOR

The cursor is advanced one position on the screen. If this puts it beyond the 40th column, then it is placed at the beginning of the next line. If the length of that line is less than 80, then this new line is linked to the previous one. A space is opened if data already exists on the new line. If the cursor has reached the bottom of the screen, then the screen is scrolled down.

.e6b6  jsr $e8b3	; check line increment

inc $d3	; increment PNTR, cursor column on current line

lda $d5	; LNMX, physical screen line length

cmp $d3	; compare to PNTR

bcs $e700	; not beyond end of line, exit

cmp #$4f	; $4f = 79

beq $e6f7	; put cursor on new logical line

lda $0292	; AUTODN, auto scroll down flag

beq $e6cd	; auto scroll is on

jmp $e967	; open a space on the screen

.e6cd  ldx $d6	; read TBLX, current line number

cpx #$19	; $19 = 25

bcc $e6da	; less than 25

jsr $e8ea	; scroll down

dec $d6	; place cursor on line 24

ldx $d6

.e6da  asl $d9,x	; clear bit7 in LDTB1 to indicate that it is line 2

lsr $d9,x	; in the logical line

inx	; next line

lda $d9,x	; set bit7 in LDTB1 to indicate that it is line 1

ora #$80	; in the logical line

sta $d9,x

dex

lda $d5	; add $28 (40) to LNMX to allow 80 characters

clc	; on the logical line

adc #$28

sta $d5

E6ED RETREAT CURSOR

The screen line link table is searched, and then the start of line is set. The rest of the routine sets the cursor onto the next line for the previous routine.

.e6ed  lda $d9,x	; LDTB1, screen line link table

bmi $e6f4	; test bit7

dex	; next line

bne $e6ed	; till all are done

.e6f4  jmp $e9f0	; set start of line

.e6f7  dec $d6	; decrement TBLX, cursor line

jsr $e87c	; goto next line

lda #$00

sta $d3	; set PNTR, the cursor column, to zero

.e700  rts

E701 BACK ON TO PREVIOUS LINE

This routine is called when using <DEL> and <cursor LEFT>. The line number is tested, and if the cursor is already on the top line, then no further action is taken. The screen pointers are set up and the cursor placed at the end of the previous line.

.e701  ldx $d6	; test TBLX, physical line number

bne $e70b	; if not on top line, branch

stx $d3	; set PNTR to zero as well

pla

pla

bne $e6a8	; allways jump

.e70b  dex	; decrement TBLX

stx $d6	; and store

jsr $e56c	; set screen pointers

ldy $d5	; get LNMX

sty $d3	; and store in PNTR

rts

E716 OUTPUT TO SCREEN

This routine is part of the main KERNAL CHROUT routine. It prints CBM ASCII characters to the screen and takes care of all the screen editing characters. The cursor is automatically updated and scrolling occurs if necessary. On entry, (A) must hold the character to be output. On entry all registers are stored on the stack. For convinience, the routine is slpit into sections showing the processing of both shifted and unshifted character.

.e716  pha	; store (A), (X) and (Y) on stack

sta $d7	; temp store

txa

pha

tya

pha

lda #$00

sta $d0	; store in CRSW

ldy $d3	; PNTR, cursor positions on line

lda $d7	; retrieve from temp store

bpl $e72a	; do unshifted characters

jmp $e7d4	; do shifted characters

UNSHIFTED CHARACTERS. Ordinary unshifted ASCII characters and PET graphics are output directly to the screen. The following control codes are trapped and precessed: <RETURN>, <DEL>, <CRSR RIGHT>, <CRSR DOWN>. If either insert mode is on or quotes are open (except for <DEL>) then the control characters are not processed, but output as reversed ASCII literals.

.e72a  cmp #$0d	; <RETURN>?

bne $e731	; nope

jmp $e891	; execute return

.e731  cmp #$20	; <SPACE>?

bcc $e745

cmp #$60	; #$60, first PET graphic character?

bcc $e73d

and #$df	; %11011111

bne $e73f

.e73d  and #$3f	; %00111111

.e73f  jsr $e684	; do quotes test

jmp $e693	; setup screen print

.e745  ldx $d8	; INSRT, insert mode flag

beq $e74c	; mode not set

jmp $e697	; output reversed charcter

.e74c  cmp #$14	; <DEL>?

bne $e77e	; nope

tya	; (Y) holds cursor column

bne $e759	; not start of line

jsr $e701	; back on previous line

jmp $e773

.e759  jsr $e8a1	; check line decrement

dey	; decrement cursor column

sty $d3	; and store in PNTR

jsr $ea24	; syncronise colour pointer

.e762  iny	; copy character at cursor position (Y+1) to (Y)

lda ($d1),y	; read character

dey

sta ($d1),y	; and store it one position back

iny

lda ($f3),y	; read character  colour

dey

sta ($f3),y	; and store it one position back

iny	; more characters to move

cpy $d5	; compare with LNMX, length of physical screen line

bne $e762	; if not equal, move more characters

.e773  lda #$20

sta ($d1),y	; store <SPACE> at end of line

lda $0286	; COLOR, current character colour

sta ($f3),y	; store colour at end of line

bpl $e7cb	; allways jump

.e77e  ldx $d4	; QTSW, editor in quotes mode

beq $e785	; no

jmp $e697	; output reversed character

.e785  cmp #$12	; <RVS>?

bne $e78b	; no

sta $c7	; RVS, reversed character output flag

.e78b  cmp #$13	; <HOME>?

bne $e792	; no

jsr $e566	; home cursor

.e792  cmp #$1d	; <CRSR RIGHT>?

bne $e7ad	; nope

iny	; increment (Y), internal counter for column

jsr $e8b3	; check line increment

sty $d3	; store (Y) in PNTR

dey	; decrement (Y)

cpy $d5	; and compare to LNMX

bcc $e7aa	; not exceeded line length

dec $d6	; TBLX, current physical line number

jsr $e87c	; goto next line

ldy #$00

.e7a8  sty $d3	; set PNTR to zero, cursor to the left

.e7aa  jmp $e6a8	; finish screen print

.e7ad  cmp #$11	; <CRSR DOWN>?

bne $e7ce	; no

clc	; prepare for add

tya	; (Y) holds cursor column

adc #$28	; add 40 to next line

tay	; to (Y)

inc $d6	; increment TBLX, physical line number

cmp $d5	; compare to LNMX

bcc $e7a8	; finish screen print

beq $e7a8	; finish screen print

dec $d6	; restore TBLX

.e7c0  sbc #$28

bcc $e7c8

sta $d3	; store PNTR

bne $e7c0

.e7c8  jsr $e87c	; go to next line

.e7cb  jmp $e6a8	; finish screen print

.e7ce  jsr $e8cb	; set colour code

jmp $ec44	; do graphics/text control

SHIFTED CHARACTERS. These are dealt with in the following order: Shifted ordinart ASCII and PET graphics characters, <shift RETURN>, <INST>, <CRSR UP>, <RVS OFF>, <CRSR LEFT>, <CLR>. If either insert mode is on, or quotes are open, then the control character is not processed but reversed ASCII literal is printed.

.e7d4  and #$7f	; clear bit7

cmp #$7f	; compare to #$7f

bne $e7dc	; not equal

lda #$5e	; if #$7f, load #$5e

.e7dc  cmp #$20	; ASCII <SPACE>?

bcc $e7e3

jmp $e691	; set up screen print

.e7e3  cmp #$0d	; <RETURN>?

bne $e7ea	; nope

jmp $e891	; do return

.e7ea  ldx $d4	; read QTSW

bne $e82d	; if quotes mode, jump

cmp #$14	; <INST>?

bne $e829	; nope

ldy $d5	; LNMX

lda ($d1),y	; get screen character

cmp #$20	; space?

bne $e7fe	; nope

cpy $d3	; PNTR equal to LNMX

bne $e805	; nope

.e7fe  cpy #$4f	; #$4f=79, last character

beq $e826	; end of logical line, can not insert

jsr $e965	; open space on line

.e805  ldy $d5	; LNMX

jsr $ea24	; syncronise colour pointer

.e80a  dey	; prepare for move

lda ($d1),y	; read character at pos (Y)

iny

sta ($d1),y	; and move one step to the right

dey

lda ($f3),y	; read character colour

iny

sta ($f3),y	; move one step to the right

dey	; decrement counter

cpy $d3	; compare with PNTR

bne $e80a	; till all characters right of cursor are moved

lda #$20	; <SPACE>, ASCII #$20

sta ($d1),y	; store at new character position

lda $0286	; COLOR, current character colour

sta ($f3),y	; store at new colour position

inc $d8	; INSRT FLAG

.e826  jmp $e6a8	; finish screen print

.e829  ldx $d8	; INSRT FLAG

beq $e832	; insert mode is off

.e82d  ora #$40

jmp $e697	; set up screen print

.e832  cmp #$11	; <CRSR UP>?

bne $e84c	; nope

ldx $d6	; read TBLX

beq $e871	; at topline, do nothing

dec $d6	; else decrement TBLX

lda $d3	; PNTR

sec	; prepare for substract

sbc #$28	; back 40 columns for double line

bcc $e847	; skip

sta $d3	; store PNTR

bpl $e871	; finish screen print

.e847  jsr $e56c	; set screen pointer

bne $e871	; finish screen print

.e84c  cmp #$12	; <RVS OFF>?

bne $e854	; nope

lda #$00

sta $c7	; RVS, disable reverse print

.e854  cmp #$1d	; <CRSR LEFT>?

bne $e86a	; nope

tya	; (Y) holds cursor column

beq $e864	; at first position

jsr $e8a1	; check line decrement

dey	; one position left

sty $d3	; store in PNTR

jmp $e6a8	; finish screen print

.e864  jsr $e701	; back to previous line

jmp $e6a8	; finish screen print

.e86a  cmp #$13	; <CLR>?

bne $e874	; nope

jsr $e544	; clear screen

.e871  jmp $e6a8	; finish screen print

.e874  ora #$80

jsr $e8cb	; set colour code

jmp $ec4f	; set graphics/text mode

E87C GO TO NEXT LINE

The cursor is placed at the start of the next logical screen line. This involves moving down two lines for a linked line. If this places the cursor below the bottom of the screen, then the screen is scrolled.

.e87c  lsr $c9	; LXSP, cursor X-Y position

ldx $d6	; TBLX, current line number

.e880  inx	; next line

cpx #$19	; 26th line

bne $e888	; nope, scroll is not needed

jsr $e8ea	; scroll down

.e888  lda $d9,x	; test LTDB1, screen line link table if first of two

bpl $e880	; yes, jump down another line

stx $d6	; store in TBLX

jmp $e56c	; set screen pointers

E891 OUTPUT <CARRIAGE RETURN>

All editor modes are swithed off and the cursor placed at the start of the next line.

.e891  ldx #$00

stx $d8	; INSRT, disable insert mode

stx $c7	; RVS, disable reversed mode

stx $d4	; QTSW, disable quotes mode

stx $d3	; PNTR, put cursor at first column

jsr $e87c	; go to next line

jmp $e6a8	; finish screen print

E8A1 CHECK LINE DECREMENT

When the cursor is at the beginning of a screen line, if it is moved backwards, this routine places the cursor at the end of the line above. It tests both column 0 and column 40.

.e8a1  ldx #$02

lda #$00

.e8a5  cmp $d3	; test if PNTR is at the first column

beq $e8b0	; yepp

clc	; add $28 (40)

adc #$28	; to test if cursor is at line two in the logical line

dex

bne $e8a5	; test two lines

rts

.e8b0  dec $d6	; decrement line number

rts

E8B3 CHECK LINE INCREMENT

When the cursor is at the end of the screen, if it is moved forward, this routine places the cursor at the start of the line below.

.e8b3  ldx #$02

lda #$27	; start by testing position $27 (39)

.e8b7  cmp $d3	; compare with PNTR

beq $e8c2	; brach if equal, and move cursor down

clc	; else, add $28 to test next physical line

adc #$28

dex	; two lines to test

bne $e8b7

rts	; return here without moving cursor down

.e8c2  ldx $d6	; get TBLX

cpx #$19	; and test if at the 25th line

beq $e8ca	; yepp, return without moving down

inc $d6	; increment TBLX

.e8ca  rts

E8CB SET COLOUR CODE

This routine is called by the output to screen routine. The Commodore ASCII code in (A) is compared with the ASCII colout code table. If a match is found, then the table offset (and hence the colour value) is stored in COLOR.

.e8cb  ldx #$0f	; 16 values to be tested

.e8cd  cmp $e8da,x	; compare with colour code table

beq $e8d6	; found, jump

dex	; next colour in table

bpl $e8cd	; till all 16 are tested

rts	; if not found, return

.e8d6  stx $0286	; if found, store code in COLOR

rts

E8DA COLOUR CODE TABLE

This is a table containing 16 Commodore ASCII codes representing the 16 available colours. Thus red is represented as $1c in the table, and would be obtained by PRINT CHR$(28), or poke 646,2.

.e8da  90	; color0, black

.e8db  05	; color1, white

.e8dc  1c	; color2, red

.e8dd  9f	; color3, cyan

.e8de  9c	; color4, purple

.e8df  1e	; color5, green

.e8e0  1f	; color6, blue

.e8e1  9e	; color7, yellow

.e8e2  81	; color8, orange

.e8e3  95	; color9, brown

.e8e4  96	; colorA, pink

.e8e5  97	; colorB, grey1

.e8e6  98	; colorC, grey2

.e8e7  99	; colorD, light green

.e8e8  9a	; colorE, light blue

.e8e9  9b	; colorF, grey3

E8EA SCROLL SCREEN

This routine scrolls the screen down by one line. If the top two lines are linked togeather, then the scroll down is repeated. The screen line link pointers are updated, each screen line is cleared and the line below is moved up. The keyboard is directly read from CIA#1, and the routine tests if <CTRL> is pressed. A JiffyDOS feature is the <CTRL S> option, which freezes the scroll till another key is pressed.

.e8ea  lda $ac	; temp store SAL on stack

pha

lda $ad

pha

lda $ae	; temp store EAL on stack

pha

lda $af

pha

.e8f6  ldx #$ff

dec $d6	; decrement TBLX

dec $c9	; decrement LXSP

dec $02a5	; temp store for line index

.e8ff  inx

jsr $e9f0	; set start of line (X)

cpx #$18

bcs $e913

lda $ecf1,x	; read low-byte screen addresses

sta $ac

lda $da,x

jsr $e9c8	; move a screen line

bmi $e8ff

.e913  jsr $e9ff	; clear a screen line

ldx #$00

.e918  lda $d9,x	; calcuate new screen line link table

and #$7f	; clear bit7

ldy $da,x

bpl $e922

ora #$80	; set bit7

.e922  sta $d9,x	; store new value in table

inx	; next line

cpx #$18	; till all 25 are done

bne $e918

lda $f1	; bottom line link

ora #$80	; unlink it

sta $f1	; and store back

lda $d9	; test top line link

bpl $e8f6	; line is linked, scroll again

inc $d6	; increment TBLX

inc $02a5

.e938  jsr $eb42	; lda #$7f, sta $dc00, rts

lda $dc01	; read keyboard decode column

cmp #$fb	; <CTRL> pressed

bne $e956	; nope, exit

ldx $c6	; NDX, number of characters in keyboard buffer

beq $e938	; freeze scroll as long as <CTRL> is pressed

lda $0276,x	; read character from keyboard buffer

sbc #$13	; substract $13, "S"

bne $e956	; nope, did not press "S"

sta $c6	; clear NDX

.e94f  cli	; allow interrupts

cmp $c6	; any new character in buffer

beq $e94f	; nope, still freeze

sta $c6	; clear NDX

.e956  ldx $d6	; read TBLX

.e958  pla	; retrieve EAL

sta $af

pla

sta $ae

pla	; retrieve SAL

sta $ad

pla

sta $ac

rts	; exit

E965 OPEN A SPACE ON THE SCREEN

This routine opens a space on the screen for use with <INST>. If needed, the screen is then scrolled down, otherwise the screen line is moved and cleared. Finally the screen line link table is adjusted and updated.

.e965  ldx $d6	; TBLX, current cursor line number

.e967  inx	; test next

lda $d9,x	; LDTB1, screen line link table

bpl $e967

stx $02a5	; temp line for index

cpx #$18	; bottom of screen

beq $e981	; yes

bcc $e981	; above bottom line

jsr $e8ea	; scroll screen down

ldx $02a5	; temp line for index

dex

dec $d6	; TBLX

jmp $e6da	; adjust link table and end

.e981  lda $ac	; push SAL, scrolling pointer

pha

lda $ad

pha

lda $ae	; push EAL, end of program

pha

lda $af

pha

ldx #$19

.e98f  dex

jsr $e9f0	; set start of line

cpx $02a5	; temp line for index

bcc $e9a6

beq $e9a6

lda $ecef,x	; screen line address table

sta $ac	; SAL

lda $d8,x	; LDTB1

jsr $e9c8	; move screen line

bmi $e98f

.e9a6  jsr $e9ff	; clear screen line

ldx #$17	; fix screen line link table

.e9ab  cpx $02a5	; temp line for index

bcc $e9bf

lda $da,x	; LDTB1+1

and #$7f

ldy $d9,x	; LDTB1

bpl $e9ba

ora #$80

.e9ba  sta $da,x

dex	; next line

bne $e9ab	; till line zero

.e9bf  ldx $02a5	; temp line for index

jsr $e6da	; adjust link table

jmp $e958	; pull SAL and EAL

E9C8 MOVE A SCREEN LINE

This routine synchronises colour transfer, and then moves the screen line pointed to down, character by character. The colour codes for each character are also moved in the same way.

.e9c8  and #$03

ora $0288	; HIBASE, top of screen page

sta $ad	; store >SAL, screen scroll pointer

jsr $e9e0	; synchronise colour transfer

ldy #$27	; offset for character on screen line

.e9d4  lda ($ac),y	; move screen character

sta ($d1),y

lda ($ae),y	; move character colour

sta ($f3),y

dey	; next character

bpl $e9d4	; till all 40 are done

rts

E9E0 SYNCHRONISE COLOUR TRANSFER

This routine setd up a temporary pointer in EAL to the colour RAM address that corresponts to the temporary screen address held in EAL.

.e9e0  jsr $ea24	; synchronise colour pointer

lda $ac	; SAL, pointer for screen scroll

sta $ae	; EAL

lda $ad

and #$03

ora #$d8	; setup colour ram to $d800

sta $af

rts

E9F0 SET START OF LINE

On entry, (X) holds the line number. The low byte of the address is set from the ROM table, and the highbyte derived from the screen link and HIBASE.

.e9f0  lda $ecf0,x	; table of screen line to bytes

sta $d1	; <PNT, current screen line address

lda $d9,x	; LDTB1, screen line link table

and #$03

ora $0288	; HIBASE, page of top screen

sta $d2	; >PNT

rts

E9FF CLEAR SCREEN LINE

The start of line is set and the screen line is cleared by filloing it with ASCII spaces. The corresponding line of colour RAM is also cleared to the value held in COLOR.

.e9ff  ldy #$27

jsr $e9f0	; set start of line

jsr $ea24	; synchronise colour pointer

.ea07  jsr $e4da	; reset character colour, to COLOR

lda #$20	; ASCII space

sta ($d1),y	; store character on screen

dey	; next

bpl $ea07	; till hole line is done

rts

nop	; free byte

EA13 PRINT TO SCREEN

The colour pointer is synchronised, and the character in (A) directly stored in the screen RAM. The character colour in (X) is stored at the equivalent point in the colour RAM.

.ea13  tay	; put print character in (Y)

lda #$02

sta $cd	; store in BLNCT, timer to toggle cursor

jsr $ea24	; synchronise colour pointer

tya	; print character back to (A)

.ea1c  ldy $d3	; PNTR, cursor column on line

sta ($d1),y	; store character on screen

txa

sta ($f3),y	; stor character colour

rts

EA24 SYNCHRONISE COLOUR POINTER

The pointer to the colour RAM is set up according to the current screen line address. This is done by reading the current screen line address and modefying it to colour RAM pointers and write it to USER at $f3/$f4

.ea24  lda $d1	; copy screen line low byte

sta $f3	; to colour RAM low byte

lda $d2	; read'n modify the hi byte

and #$03

ora #$d8

sta $f4	; to suite the colour RAM

rts

EA31 MAIN IRQ ENTRY POINT

This routine services the normal IRQ that jumps through the hardware vector to $ff48, and then continues to the CINV vector at $0314. First it checks if the <STOP> key was pressed and updates the realtime clock. Next, the cursor is updated (if it is enabled, BLNSW). The blink counter, BLNCT, is decremented. When this reaches zero, the cursor is toggled (blink on/off). Finally it scans the keyboard. The processor registers are then restored on exit.

Area from $ea64 to $ea7b has been changed in the JiffyDOS system. Some routintes to handle the casetterecorder has been removed.

.ea31  jsr $ffea	; update realtime clock, routine UDTIM

lda $cc	; read BLNSW to see if cursor is enabled

bne $ea61	; nope

dec $cd	; read BLNCT

bne $ea61	; if zero, toggle cursor - else jump

lda #$14	; blink speed

sta $cd	; restore BLCNT

ldy $d3	; get PNTR, cursor column

lsr $cf	; BLNON, flag last cursor blink on/off

ldx $0287	; get background colour under cursor, GDCOL

lda ($d1),y	; get screen character

bcs $ea5c	; ?

inc $cf	; increment BLNON

sta $ce	; temporary store character under cursor

jsr $ea24	; synchronise colour pointer

lda ($f3),y	; get colour under character

sta $0287	; store in GDCOL

ldx $0286	; get current COLOR

lda $ce	; retrieve character under cursor

.ea5c  eor #$80	; toggle cursor by inverting character

jsr $ea1c	; print to screen by using part of 'print to screen'

.ea61  jmp $ea7b	; skip

EA64 JIFFYDOS CRNCH

The ICRNCH VECTOR points to this routine after the JiffyDOS init.

.ea64  pla	; get last stack entry

pha	; put back

cmp #$98	; equal to #$98

beq $ea6d	; yepp, do JiffyDOS CRNCH

.ea6a  jmp $a57c	; jump to original CRNCH

.ea6d  jsr $f72c	; test if key in buffer	is a JiffyDOS command

bne $ea6a	; no command

ldx $7a	; position i keybordbuffer

ldy #$04	; setup values for old routine

tya

jmp $a5e3	; back into old CRNCH

byte .xxx	; free byte?? serialnumber!!

EA7B QUICK IRQ ENTRY POINT

If you dont want the screenupdate, of if you take care of it yourself, you can use this quick exit, specially $ea81.

.ea7b  jsr $ea87	; scan keyboard

lda $dc0d	; clear CIA#1 I.C.R to enable next IRQ

.ea81  pla	; restore (Y), (X), (A)

tay

pla

tax

pla

rti	; back to normal

EA87 SCNKEY: SCAN KEYBOARD

The KERNAL routine SCNKEY ($ff9f) jumps to this routine. First, the shift-flag, SHFLAG, is cleared, and the keyboard tested for nokey. The keyboard is set up as a 8 * 8 matrix, and is read one row at a time. $ff indicates that no key has been pressed, and a zerobit, that one key has been pressed.

.ea87  lda #$00

sta $028d	; clear SHFLAG

ldy #$40

sty $cb

sta $dc00	; store in keyboard write register

ldx $dc01	; keyboard read register

cpx #$ff	; no key pressed

beq $eafb	; skip

tay

lda #$81	; point KEYTAB vector to $eb81

sta $f5

lda #$eb

sta $f6

lda #$fe	; bit0 = 0

sta $dc00	; will test first row in matrix

.eaa8  ldx #$08	; scan 8 rows in matrix

pha	; temp store

.eaab  lda $dc01	; read

cmp $dc01	; wait for value to settle (key bouncing)

bne $eaab

.eab3  lsr a	; test bit0

bcs $eacc	; no key pressed

pha

.eab7  lda ($f5),y	; get key from KEYTAB

cmp #$05	; value less than 5

bcs $eac9	; nope

cmp #$03	; value = 3

beq $eac9	; nope

ora $028d

sta $028d	; store in SHFLAG

bpl $eacb

.eac9  sty $cb	; store keynumber we pressed in SFDX

.eacb  pla

.eacc  iny	; key counter

cpy #$41	; all 64 keys (8*8)

bcs $eadc	; jump if ready

dex	; next key in row

bne $eab3	; row ready

sec	; prepare for rol

pla

rol a	; next row

sta $dc00	; store bit

bne $eaa8	; always jump

.eadc  pla	; clean up

EADD PROCESS KEY IMAGE

This routine decodes the pressed key, and calcuates its ASCII value, by use of the four tables. If the pressed key is the same key as in the former interrupt, then the key-repeat-section is entered. The routine tests the RPTFLG if the key shall repeat. The new key is stored in the keyboard buffer, and all pointers are uppdated.

.eadd  jmp ($028f)	; jump through KEYLOG vector, points to $eae0

.eae0  ldy $cb	; SFDX, number of the key we pressed

lda ($f5),y	; get ASCII value from decode table

tax	; temp store

cpy $c5	; same key as former interrupt

beq $eaf0	; yepp

ldy #$10	; restore the repeat delay counter

sty $028c	; DELAY

bne $eb26	; always jump

.eaf0  and #$7f

bit $028a	; RPTFLG, test repeat mode

bmi $eb0d	; repeat all keys

bvs $eb42	; repeat none - exit routine

cmp #$7f

.eafb  beq $eb26

cmp #$14	; <DEL> key pressed

beq $eb0d	; yepp...

cmp #$20	; <space> key pressed

beq $eb0d	; yepp...

cmp #$1d	; <CRSR LEFT/RIGHT>

beq $eb0d	; yepp..

cmp #$11	; <CRSRS DOWN/UP>

bne $eb42	; yepp..

.eb0d  ldy $028c	; DELAY

beq $eb17	; skip

dec $028c	; decrement DELAY

bne $eb42	; end

.eb17  dec $028b	; decremant KOUNT, repeat speed counter

bne $eb42	; end

ldy #$04

sty $028b	; init KOUNT

ldy $c6	; read NDX, number of keys in keyboard queue

dey

bpl $eb42	; end

.eb26  ldy $cb	; read SFDX

sty $c5	; store in LSTX

ldy $028d	; read SHFLAG

sty $028e	; store in LSTSHF, last keyboard shift pattern

cpx #$ff	; no valid key pressed

beq $eb42	; end

txa

ldx $c6	; NDX, number of keys in buffer

cpx $0289	; compare to XMAX, max numbers oc characters in buffer

bcs $eb42	; buffer is full, end

sta $0277,x	; store new character in keyboard buffer

inx	; increment counter

stx $c6	; and store in NDX

.eb42  lda #$7f

sta $dc00	; keyboard write register

rts	; exit

.eb48  lda $028d	; SHFLAG

cmp #$03	; <SHIFT> and <CBM> at the same time

bne $eb64	; nope

cmp $028e	; same as LSTSHF

beq $eb42	; if so, end

lda $0291	; read MODE, shift key enable flag

bmi $eb76	; end

lda $d018	; VIC memory control register

eor #$02	; toggle character set, upper/lower case

sta $d018	; and store

jmp $eb76	; process key image

.eb64  asl a

cmp #$08	; test <CTRL>

bcc $eb6b	; nope

lda #$06	; set offset for ctrl

.eb6b  tax	; to (X)

lda $eb79,x	; read keyboard select vectors, low byte

sta $f5	; store in KEYTAB, decode table vector

lda $eb7a,x	; read keyboard select vectors, high byte

sta $f6	; KEYTAB+1

.eb76  jmp $eae0	; process key image

EB79 KEYBOARD SELECT VECTORS

This is a table of vectors pointing to the start of the four keyboard decode tables.

.eb79  81 eb	; vector to unshifted keyboard, $eb81

.eb7b  c2 eb	; vector to shifted keyboard, $ebc2

.eb7d  03 ec	; vector to cbm keyboard, $ec03

.eb7f  78 ec	; vector to ctrl keyboard, $ec78

EB81 KEYBOARD 1 - UNSHIFTED

This is the first of four keybboard decode tables. The ASCII code for the key pressed is at the intersection of the row (written to $dc00) and the column (read from $dc01). The matrix values are shown below. Note that left and right shift keys are seperated.

.eb81  14 0d 1d 88 85 86 87 11

.eb89  33 57 41 34 5a 53 45 01

.eb91  35 52 44 36 43 46 54 58

.eb99  37 59 47 38 42 48 55 56

.eba1  39 49 4a 30 4d 4b 4f 4e

.eba9  2b 50 4c 2d 2e 3a 40 2c

.ebb1  5c 2a 3b 13 01 3d 5e 2f

.ebb9  31 5f 04 32 20 02 51 03

.ebc1  ff	; free byte

| DEL | RETURN | CRSR RI | F7 | F1 | F3 | F5 | CRSR DO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | w | a | 4 | z | s | e | LE SHIFT |
| 5 | r | d | 6 | c | f | t | x |
| 6 | y | g | 8 | b | h | u | v |
| 9 | i | j | 0 | m | k | o | n |
| + | p | l | - | . | : | @ | , |
| £ | * | ; | HOME | RI SHIFT | = | ^¦ | / |
| 1 | <- | CTRL | 2 | SPACE | CBM | q | STOP |

EBC2 KEYBOARD 2 - SHIFTED

This is the second of four keyboard decode tables. The ASCII code for the key pressed is at the intersection of the row (written to $dc00) and the column (read from $dc01). The matrix values are shown below.

.ebc2  94 8d 9d 8c 89 8a 8b 91

.ebca  23 d7 c1 24 da d3 c5 01

.ebd2  25 d2 c4 26 c3 c6 d4 d8

.ebda  27 d9 c7 28 c2 c8 d5 d6

.ebe2  29 c9 ca 30 cd cb cf ce

.ebea  db d0 cc dd 3e 5b ba 3c

.ebf2  a9 c0 5d 93 01 3d de 3f

.ebfa  21 5f 04 22 a0 02 d1 83

.ec02  ff	; free byte

| INST | RRETURN | CRSR LE | F8 | F2 | F4 | F6 | CRSR UP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # | W | A | $ | Z | S | E | LE SHIFT |
| % | R | D | & | C | F | T | X |
| ' | Y | G | ( | B | H | U | V |
| ) | I | J | 0 | M | K | O | N |
| cbm gr | P | L | cbm gr | > | [ | cbm gr | < |
| cbm gr | cbm gr | [ | CLR | RI SHIFT | = | pi | ? |
| ! | <- | CTRL | " | SPACE | CBM | Q | RUN |

EC03 KEYBOARD 3 - COMMODORE

This is the third of four keyboard decode tables. The ASCII code for the key pressed is at the intersection of the ro (written to $dc00) and hte column (read from $dc01). The matrix values are shown below.

.ec03  94 8d 9d 8c 89 8a 8b 91

.ec0b  96 b3 b0 97 ad ae b1 01

.ec13  98 b2 ac 99 bc bb a3 bd

.ec1b  9a b7 a5 9b bf b4 b8 be

.ec23  29 a2 b5 30 a7 a1 b9 aa

.ec2b  a6 af b6 dc 3e 5b a4 3c

.ec33  a8 df 5d 93 01 3d de 3f

.ec3b  81 5f 04 95 a0 02 ab 83

.ec43  ff	; free byte

| INST | RETURN | CRSR LE | F8 | F2 | F4 | F6 | CRSR UP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pink | cbm gr | cbm gr | grey 1 | cbm gr | cbm gr | cbm gr | LE SHIFT |
| grey 2 | cbm gr | cbm gr | li green | cbm gr | cbm gr | cbm gr | cbm gr |
| li blue | cbm gr | cbm gr | grey 3 | cbm gr | cbm gr | cbm gr | cbm gr |
| ) | cbm gr | cbm gr | 0 | cbm gr | cbm gr | cbm gr | cbm gr |
| cbm gr | cbm gr | cbm gr | cbm gr | > | [ | cbm gr | < |
| cbm gr | cbm gr | ] | CLR | RI SHIFT | = | pi | ? |
| orange | <- | CTRL | brown | SPACE | CBM | cbm gr | RUN |

EC44 GRAPHICS / TEXT CONTROL

This routine is used to toggle between text and graphics character set, and to enable/disable the <shift-CBM> keys. The routine is called by the main 'output to screen' routine, and (A) holds a CBM ASCII code on entry.

.ec44  cmp #$0e	; <switch to lower case>

bne $ec4f	; nope

lda $d018	; VIC memory control register

ora #$02	; set bit1

bne $ec58	; allways branch

.ec4f  cmp #$8e	; <switch to upper case>

bne $ec5e	; nope

lda $d018	; VIC memory control register

and #$fd	; clear bit1

.ec58  sta $d018	; and store

.ec5b  jmp $e6a8	; finish screen print

.ec5e  cmp #$08	; <disable <shift-CBM>>

bne $ec69	; nope

lda #$80

ora $0291	; disable MODE

bmi $ec72	; allways jump

.ec69  cmp #$09	; <enable <shift-CBM>>

bne $ec5b	; nope, exit

lda #$7f

and $0291	; enable MODE

.ec72  sta $0291	; store MODE, enable/disable shift keys

jmp $e6a8	; finish screen print

EC78 KEYBOARD 4 - CONTROL

This is the last keyboard decode table. The ASCII code for the key pressed is at the intersection of the row (written to $dc00) and the column (read from $dc01). The matrix values are shown below.

A few special funktion are found in this table ie.

<ctrl H> - disables the upper/lower case switch

<ctrl I> - enables the upper/lower case switch

<ctrl S> - homes the cursor

<ctrl T> - delets character

Note that the italic keys only represent a ASCII code, and not a CBM character.

Future implementations: Change some of the $ff values which represents 'no key' to a valid ASCII code. ESC ($1b) and why not use the F-keys for something useful.

.ec78  ff ff ff ff ff ff ff ff

.ec80  1c 17 01 9f 1a 13 05 ff

.ec88  9c 12 04 1e 03 06 14 18

.ec90  1f 19 07 9e 02 08 15 16

.ec98  12 09 0a 92 0d 0b 0f 0e

.eca0  ff 10 0c ff ff 1b 00 ff

.eca8  1c ff 1d ff ff 1f 1e ff

.ecb0  90 06 ff 05 ff ff 11 ff

.ecb8  ff	; free byte

| red | W | A | cyan | Z | HOME | white |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| purple | RVS ON | D | green | STOP | F | DEL | X |
| blue | Y | G | yellow | CBM | DISABLE | U | V |
| RVS ON | ENABLE | J | RVS OFF | RETURN | K | O | LOWER |
|  | P | L |  |  | ] | @ |  |
| red |  | CRSR RI |  |  | blue | green |  |
|  | <- |  | white |  |  | CRSR DO |  |

ECB9 VIDEO CHIP SET UP TABLE

This is a table of the initial values for the VIC chip registers at start up.

.ecb9  00 00	; $d000/1, sprite0 - x,y cordinate

.ecbb  00 00	; $d002/3, sprite1 - x,y cordinate

.ecbd  00 00	; $d004/5, sprite2 - x,y cordinate

.ecbf  00 00	; $d006/7, sprite3 - x,y cordinate

.ecc1  00 00	; $d008/9, sprite4 - x,y cordinate

.ecc3  00 00	; $d00a/b, sprite5 - x,y cordinate

.ecc5  00 00	; $d00c/d, sprite6 - x,y cordinate

.ecc7  00 00	; $d00e/f, sprite7 - x,y cordinate

.ecc9  00	; $d010, sprite MSB

.ecca  9b	; $d011, VIC control register

.eccb  37	; $d012,

.eccc  00 00	; $d013/4, light pen x/y position

.ecce  00	; $d015, sprite enable

.eccf  08	; $d016, VIC control register 2

.ecd0  00	; $d017, sprite y-expansion

.ecd1  14	; $d018, VIC memory control register

.ecd2  0f	; $d019, VIC irq flag register

.ecd3  00	; $d01a, VIC irq mask register

.ecd4  00	; $d01b, sprite/background priority

.ecd5  00	; $d01c, sprite multicolour mode

.ecd6  00	; $d01d, sprite x-expansion

.ecd7  00	; $d01e, sprite/sprite collision

.ecd8  00	; $d01f, sprite/background collision

.ecd9  0e	; $d020, border colour (light blue)

.ecda  06	; $d021, background colour 0 (blue)

.ecdb  01	; $d022, background colour 1

.ecdc  02	; $d023, background colour 2

.ecdd  03	; $d024, background colour 3

.ecde  04	; $d025, sprite multicolour register 0

.ecdf  00	; $d026, sprite multicolour register 1

.ece0  01	; $d027, sprite0 colour

.ece1  02	; $d028, sprite1 colour

.ece2  03	; $d029, sprite2 colour

.ece3  04	; $d02a, sprite3 colour

.ece4  05	; $d02b, sprite4 colour

.ece5  06	; $d02c, sprite5 colour

.ece6  07	; $d02d, sprite6 colour

ECE7 SHIFT-RUN EQUIVALENT

This is the message LOAD <CR> RUN <CR>, which is placed in the keyboard buffer when <shift-RUN> is pressed.

.ece7  4c 4f 41 44 0d	; LOAD <CR>

.ecec  52 55 4e 0d	; RUN <CR>

ECF0 LOW BYTE SCREEN LINE ADDRESSES

This is a table of the low bytes of screen line addresses. The high byte of the addresses is obtained by derivation from the page on which the screen starts. There was an additional table of high byte addresses on the fixed screen PETs.

.ecf0  00 28 50 78 a0

.ecf5  c8 f0 18 40 68

.ecfa  90 b8 e0 08 30

.ecff  58 80 a8 d0 f8

.ed04  20 48 70 98 c0

ED09 TALK: SEND 'TALK' / 'LISTEN'

The KERNAL routine TALK ($ffb4) and LISTEN ($ffb1) are vectored here. The routine sends the command 'TALK' or 'LISTEN' on the serial bus. On entry (A) must hold the device number to which the command will be sent. The two entry points differ only in that to TALK, (A) is ORed with #$40, and to LISTEN, (A) is ORed with #$20. The UNTALK (#$3f) and UNLISTEN (#$5f) are also sent via this routine, but their values are set on entry. If there is a character waiting to go out on the bus, then this is output. Handshaking is performed, and ATN (attension) is set low so that the byte is interpreted as a command. The routine drops through to the next one to output the byte on the serial bus. Note that on conclusion, ATN must be set high.

.ed09  ora #$40	; set TALK flag

.byte $2c	; bit $2009, mask ORA command

ora #$20	; set LISTEN flag

jsr $f0a4	; check serial bus idle

.ed11  pha

bit $94	; C3PO, character in serial buffer

bpl $ed20	; nope

sec	; prepare for ROR

ror $a3	; temp data area

jsr $fbfe	; JiffyDOS, send data to serial bus

lsr $94	; 3CPO

lsr $a3

.ed20  pla

sta $95	; BSOUR, buffered character for bus

sei

jsr $f0ed	; JiffyDOS, set data 1, and clear serial bit count

cmp #$3f	; UNTALK?

bne $ed2e	; nope

jsr $ee85	; set CLK 1

.ed2e  lda $dd00	; serial bus I/O port

ora #$08	; clear ATN, prepare for command

sta $dd00	; store

.ed36  sei	; disable interrupts

jsr $ee8e	; set CLK 1

jsr $ee97	; set data 1

jsr $eeb3	; delay 1 ms

ED40 SEND DATA ON SERIAL BUS

The byte of data to be output on the serial bus must have been previously stored in the serial buffer, BSOUR. An initial test is made for bus activity, and if none is detected then ST is set to #$80, ie. ?DEVICE NOT PRESENT. The byte is output by rotating it right and sending the state of the carry flag. This is done eight times until the whole byte was sent. The CIA timer is set to 65 ms and the bus is checked for 'data accepted'. If timeout occurs before this happens then ST is set to #$03, ie. write timeout. The routine is modified with a jump to $f8ea where a test is done to see if this device is a JiffyDOS device. The result is stored in $a3.

sei	; disable interrupts

jsr $ee97	; set data 1

jsr $eea9	; get serial in and clock

bcs $edad	; no activity, device not present.

jsr $ee85	; set CLK 1

bit $a3	; temp data area

bpl $ed5a

.ed50  jsr $eea9	; get serial in and clock

bcc $ed50	; wait for indata = 0

.ed55  jsr $eea9	; get serial in and clock

.ed58  bcs $ed55	; wait for indata = 1

.ed5a  jsr $eea9	; get serial in and clock

bcc $ed5a	; wait for indata = 0

jsr $ee8e	; set CLK 0

txa	; transfer (X) to (A)

pha	; store (A) on stack

ldx #$08	; output 8 bits

.ed66  pha

pla

bit $dd00	; serial bus I/O port

bmi $ed72	; no timeout

pla	; retrieve (A)

tax	; and restore (X)

jmp $edb0	; exit with flag write timeout

.ed72  jsr $ee97	; serial output 1

ror $95	; BSOUR, buffered character for bus

bcs $ed7c	; prepare to output 1

jsr $eea0	; else, serial output 0

.ed7c  jsr $ee85	; set CLK 1

lda $dd00	; serial bus I/O port

and #$df	; set data 1

ora #$10	; set CLK 0

php

pha

jsr $f8ea	; test if device on serial bus is a JiffyDOS device

pla

plp

dex	; decrement bit counter

bne $ed66	; next bit till all 8 are done

pla

tax

lda #$04

sta $dc07	; CIA timer B, high byte

lda #$19

sta $dc0f	; set 1 shot, load and start CIA timer B

lda $dc0d	; CIA ICR

.ed9f  lda $dc0d

and #$02	; timeout

bne $edb0	; yep, flag write timeout

jsr $eea9	; get serial in and clock

bcs $ed9f

cli	; enable interrupts

rts

EDAD FLAG ERRORS

(A) is loaded with one of the two error flags, depending on the entry point. #$80 signifies the device was not present, and #$03 signifies a write timeout. The value is then set into the I/O status word, ST. The routine exits by clearing ATN and giving the final handshake.

.edad  lda #$80	; flag ?DEVICE NOT PRESENT

.byte $2c	; mask LDA #$03

.edb0  lda #$03	; flag write timeout

.edb2  jsr $fe1c	; set I/O status word

cli

clc

bcc $ee03	; allways jump, do final handshake

EDB9 SECOND: SEND LISTEN SA

The KERNAL routine SECOND ($ff93) is vectored here. On entry, (A) holds the secondary address. This is placed in the serial buffer and sent to the serial bus "under attension". Finally the routine drops through to the next routine to set ATN false.

.edb9  sta $95	; store (A) in BSOUT, buffer for the serial bus

jsr $ed36	; handshake and send byte.

EDBE CLEAR ATN

The ATN, attension, line on the serial bus is set to 1, ie. ATN is now false and data sent on the serial bus will not be interpreted as a command.

.edbe  lda $dd00	; serial bus I/O port

and #$f7	; clear bit4, ie. ATN 1

sta $dd00	; store to port

rts

EDC7 TKSA: SEND TALK SA

The KERNAL routine TKSA ($ff96) is vectored here. On entry, (A) holds the secondary address. This is placed in the serial buffer and sent out to the serial bus "under attension". The routine drops through to the next routine to wait for CLK and clear ATN.

sta $95	; BSOUR, the serial bus buffer

jsr $ed36	; handshake and send byte to the bus

EDCC WAIT FOR CLOCK

This routine sets data = 0, ATN = 1 and CLK = 1. It then waits to recieve CLK = 0 from the serial bus.

sei	; disable interrupts

jsr $eea0	; set data 0

jsr $edbe	; set ATN 1

jsr $ee85	; set CLK 1

.edd6  bit $dd00	; read serial bus I/O port

bvs $edd6	; test bit6, and wait for CLK = 0

cli	; enable interrupt

rts

EDDD CIOUT: SEND SERIAL DEFERRED

The KERNAL routine CIOUT ($ffa8) jumps to this routine. If there is a character awaiting output in the buffer, then it is sent on the bus to the new JiffyDOS send routine. The output flag, C3PO is set (ie. bit 7 = 1) and the contents of (A) is placed in the serial buffer.

.eddd  bit $94	; C3PO flag, character in serial buffer

bmi $ede6	; yes

sec	; prepare for ROR

ror $94	; set C3PO

bne $edeb	; always jump

.ede6  pha	; temp store

jsr $fbfe	; JiffyDOS send data to serial bus

pla

.edeb  sta $95	; store character in BSOUR

clc	; clear carry to indicate no errors

rts

EDEF UNTLK: SEND 'UNTALK'/'UNLISTEN'

The KERNAL routine UNTALK ($ffab)and UNLISTEN ($ffae) are vectored here. ATN is set to 0, and CLK is set to 0. (A) is loaded with #$5f for 'UNTALK' and #$3f for 'UNLISTEN'. The command is sent to the serial bus via the 'send TALK/LISTEN' routine. Finally ATN is set to 1, and after s short delay, CLK and data are both set to 1.

.edef  sei	; disable interrupts

lda $dd00	; serial bus I/O

ora #$08	; set bit4

sta $dd00	; and store, set ATN 0

jsr $ee8e	; set CLK 0

lda #$5f	; flag UNTALK

.byte $2c	; mask LDA #$3f with BIT $3fa9

lda #$3f	; flag UNLISTEN

jsr $ed11	; send command to serial bus

.ee03  jsr $edbe 	; clear ATN

.ee06  txa

ldx #$0a	; init delay

.ee09  dex	; decrement counter

bne $ee09	; till ready

tax

jsr $ee85	; set CLK 1

jmp $ee97 	; set data 1

EE13 ACPTR: RECIEVE FROM SERIAL BUS

The KERNAL routine ACPTR ($ffa5) points to this routine in the original Commodore KERNAL. JiffyDOS uses a routine at $fbaa, which is the new ACPTR pointer. This routine is used when a device is not JiffyDOS equiped. A timing loop is enteredusing the CIA timer, and if a byte is not received in 65 ms, ST is set to #$02, ie. a read timeout. A test is made for EOI and if this occurs, ST is set to #$40, indicating end of file. The byte is then received from the serial bus and built up bit by bit in the temporary stora at #$a4. This is transfered to (A) on exit, unless EOI has occured.

.ee13  jmp $fbaa	; Jump to JiffyDOS ACPTR, return if no JiffyDOS device

sta $a5	; CNTDN, counter

jsr $ee85	; set CLK 1

.ee1b  jsr $eea9	; get serial in and clock

bpl $ee1b	; wait for CLK = 1

.ee20  lda #$01

sta $dc07	; setup CIA#1 timer B, high byte

lda #$19

sta $dc0f	; set 1 shot, load and start CIA timer B

jsr $ee97	; set data 1

lda $dc0d

.ee30  lda $dc0d	; read CIA#1 ICR

and #$02 	; test if timer B reaches zero

bne $ee3e	; timeout

jsr $eea9	; get serial in and clock

bmi $ee30	; CLK 1

bpl $ee56 	; CLK 0

.ee3e  lda $a5	; CNTDN

beq $ee47

lda #$02	; flag read timeout

jmp $edb2	; set I/O status word

.ee47  jsr $eea0	; set data 1

jsr $ee85	; set CLK 1

lda #$40	; flag EOI

jsr $fe1c	; set I/O status word

inc $a5	; increment CNTDN, counter

bne $ee20	; again

.ee56  lda #$08 	; set up CNTDN to receive 8 bits

sta $a5

.ee5a  lda $dd00	; serial bus I/O port

cmp $dd00	; compare

bne $ee5a	; wait for serial bus to settle

asl

bpl $ee5a	; wait for data in =1

ror $a4	; roll in received bit in temp data area

.ee67  lda $dd00	; serial bus I/O port

cmp $dd00	; compare

bne $ee67	; wait for bus to settle

asl

bmi $ee67	; wait for data in =0

dec $a5	; one bit received

bne $ee5a	; repeat for all 8 bits

jsr $eea0	; set data 1

bit $90	; STATUS, I/O status word

bvc $ee80	; not EOI

jsr $ee06	; handshake and exit without byte

.ee80  lda $a4	; read received byte

cli	; enable interrupts

clc	; clear carry, no errors

rts

EE85 SERIAL CLOCK ON

This routine sets the clock outline on the serial bus to 1. This means writing a 0 to the port. This value is reversed by hardware on the bus.

.ee85  lda $dd00	; serial port I/O register

and #$ef	; clear bit4, ie. CLK out =1

sta $dd00	; store

rts

EE8E SERIAL CLOCK OFF

This routine sets the clock outline on the serial bus to 0. This means writing a 1 to the port. This value is reversed by hardware on the bus.

.ee8e  lda $dd00	; serial port I/O register

ora #$10	; set bit4, ie. CLK out =0

sta $dd00	; store

rts

EE97 SERIAL OUTPUT 1

This routine sets the data out line on the serial bus to 1. This means writing a 0 to the port. This value is reversed by hardware on the bus.

.ee97  lda $dd00	; serial bus I/O register

and #$df	; clear bit5

sta $dd00	; store

rts

EEA0 SERIAL OUTPUT 0

This routine sets the data out line on the serial bus to 0. This means writing a 1 to the port. This value is reversed by hardware on the bus.

.eea0  lda $dd00	; serial bus I/O resister

ora #$20	; set bit 5

sta $dd00	; store

rts

EEA9 GET SERIAL DATA AND CLOCK IN

The serial port I/O register is stabilised and read. The data is shifteed into carry and CLK into bit 7. This way, both the data and clock can bee determined by flags in the processor status register. Note that the values read are true, and do not nead to be reversed in the same way as the outuput line do.

.eea9  lda $dd00	; serial port I/O register

cmp $dd00	; compare

bne $eea9	; wait for bus to settle

asl	; shift data into carry, and CLK into bit 7

rts

EEB3 DELAY 1 MS

This routine is a software delay loop where (X) is used as counter, and are decremented for a period of 1 millisecond. The original (X) is stored on entry and (A) is messed up.

.eeb3  txa	; move (X) to (A)

ldx #$b8	; start value

.eeb6  dex	; decrement

bne $eeb6	; untill zero

tax	; (A) to (X)

rts

EEBB RS232 SEND

This routine is concerned with sending a byte on the RS232 port. The data is actually written to the port under NMI interrupt control. The CTS line generates an NMI when the port is ready for data. If all the bits in the byte have been sent, then a new RS232 byte is set up. Otherwise, this routine calculates parity and number of stop bits set up in the OPEN command. These bits are added to the end of the byte being sent.

.eebb  lda $b4	; BITTS, RS232 out bit count

beq $ef06	; send new RS232 byte

bmi $ef00

lsr $b6	; RODATA, RS232 out byte buffer

ldx #$00

bcc $eec8

dex

.eec8  txa

eor $bd	; ROPRTY, RS232 out parity

sta $bd

dec $b4	; BITTS

beq $eed7

.eed1  txa

and #$04

sta $b5	; NXTBIT, next RS232 bit to send

rts

.eed7  lda #$20

bit $0294	; M51CDR, 6551 command register immage

beq $eef2	; no patity

bmi $eefc	; mark/space transmit

bvs $eef6	; even parity

lda $bd	; ROPRTY, out parity

bne $eee7

.eee6  dex

.eee7  dec $b4	; BITTS, out bit count

lda $0293	; M51CTR, 6551 control register image

bpl $eed1	; one stop bit only

dec $b4	; BITTS

bne $eed1

.eef2  inc $b4	; BITTS

bne $eee6

.eef6  lda $bd	; ROPRTY

beq $eee7

bne $eee6

.eefc  bvs $eee7

bvc $eee6

.ef00  inc $b4	; BITTS

ldx #$ff

bne $eed1

EF06 SEND NEW RS232 BYTE

This routine sets up the system variables ready to send a new byte to the RS232 port. A test is made for 3-line or X-line modus. In X-line mode, DSR and  CTS are checked.

.ef06  lda $0294	; M51CDR, 6551 command register

lsr a	; test handshake mode

bcc $ef13	; 3-line mode (no handshake)

bit $dd01	; RS232 port

bpl $ef2e	; no DSR, error

bvc $ef31	; no CTS, error

.ef13  lda #$00

sta $bd	; ROPRTY, RS232 out parity

sta $b5	; NXTBIT, next bit to send

ldx $0298	; BITNUM, number of bits left to send

stx $b4	; BITTS, RS232 out bit count

ldy $029d	; RODBS, start page of out buffer

cpy $029e	; RODBE, index to end if out buffer

beq $ef39	; disable timer

lda ($f9),y	; RS232 out buffer

sta $b6	; RODATA, RS232 out byte buffer

inc $029d	; RODBS

rts

EF2E NO DSR / CTS ERROR

(A) is loaded with the error flag - $40 for no DSR, and $10 for no CTS. This is then ORed with 6551 status image and stored in RSSTAT.

.ef2e  lda #$40	; entrypoint for 'NO DSR'

.byte $2c	; mask next LDA-command

.ef31  lda #$10	; entrypoint for 'NO CTS'

ora $0297	; RSSTAT, 6551 status register image

sta $0297

EF39 DISABLE TIMER

This routine set the interrupt mask on CIA#2 timer B. It also clears the NMI flag.

.ef39  lda #$01

.ef3b  sta $dd0d	; CIA#2 interrupt control register

eor $02a1	; ENABL, RS232 enables

ora #$80

sta $02a1	; ENABL

sta $dd0d	; CIA#2 interrupt control register

rts

EF4A COMPUTE BIT COUNT

This routine computes the number of bits in the word to be sent. The word length information is held in biits 5 & 6 of M51CTR. Bit 7 of this register indicates the number of stop bits. On exit, the number of bits is held in (X).

.ef4a  ldx #$09

lda #$20

bit $0293	; M51CTR, 6551 control register image

beq $ef54

dex

.ef54  bvc $ef58

dex

dex

.ef58  rts

EF59 RS232 RECEIVE

This routine builds up the input byte from the RS232 port in RIDATA. Each bit is input from the port under NMI interrupt control. The bit is placed in INBIT before being passed to this routine, where it is shifted into the carry flag and then rotated into RIDATA. The bit count is decremented and parity updated.

.ef59  ldx $a9	; RINONE, check for start bit?

bne $ef90

dec $a8	; BITC1, RS232 in bit count

beq $ef97	; process received byte

bmi $ef70

lda $a7	; INBIT, RS232 in bits

eor $ab	; RIPRTY, RS232 in parity

sta $ab

lsr $a7	; INBIT, put input bit into carry

ror $aa	; RIDATA,

.ef6d  rts

.ef6e  dec $a8	; BITC1

.ef70  lda $a7	; INBIT

beq $efdb

lda $0293	; M51CTR, 6551 control register image

asl a

lda #$01

adc $a8	; BITC1

bne $ef6d	; end

EF7E SET UP TO RECEIVE

This routine sets up the I.C.R. to wait for the receiver edge, and flags this into ENABL. It then flags the check for a start bit.

.ef7e  lda #$90

sta $dd0d	; CIA#2 I.C.R.

ora $02a1	; ENABL, RS232 enables

sta $02a1

sta $a9	; RINONE, check for start bit

lda #$02

jmp $ef3b	; disable timer and exit

EF90 PROCESS RS232 BYTE

The byte recieved from the RS232 port is checked against parity. This involvs checking the input parity options selected, and then verifying the parity bit calculated against that input. If the test is passed, then the byte is stored in the in-buffer. Otherwise an error is flagged into RSSTAT.

A patch in KERNAL version 3, has been added to the input routine at $ef94 to initialise the RS232 parity byte, RIPRTY, on reception of a start bit.

.ef90  lda $a7	; INBIT, RS232 in bits

bne $ef7e	; set up to receive

jmp $e4d3	; patch, init parity byte

.ef97  ldy $029b	; RIDBE, index to the end of in buffer

iny

cpy $029c	; RIDBS, start page of in buffer

beq $efca	; receive overflow error

sty $029b	; RIDBE

dey

lda $aa	; RIDATA, RS232 in byte buffer

ldx $0298	; BITNUM, number of bits left to send

.efa9  cpx #$09	; full word to come?

beq $efb1	; yes

lsr a

inx

bne $efa9

.efb1  sta ($f7),y	; RIBUF, RS232 in buffer

lda #$20

bit $0294	; M51CDR, 6551 command register image

beq $ef6e	; parity disabled

bmi $ef6d	; parity check disabled, TRS

lda $a7	; INBIT, parity check

eor $ab	; RIPRTY, RS232 in parity

beq $efc5	; receive parity error

bvs $ef6d

.byte $2c	; mask

.efc5  bvc $ef6d

lda #$01	; receive parity error

.byte $2c	; mask

.efca  lda #$04	; receive overflow

.byte $2c	; mask

.efcd  lda #$80	; framing break

.byte $2c	; mask

.efd0  lda #$02	; framing error

ora $0297	; RSSTAT, 6551 status register image

sta $0297

jmp $ef7e	; set up to receive

.efdb  lda $aa	; RIDATA

bne $efd0	; framing error

beq $efcd	; receive break

EFE1 SUBMIT TO RS232

This routine is called when data is required from the RS232 port. Its funktion is to perform the handshaking on the poort needed to receive the data. If 3 line mode is used, then no handshaking is implemented and the routine exits.

.efe1  sta $9a	; DFLTO, default output device

lda $0294	; M51CDR, 6551 command register image

lsr a

bcc $f012	; 3 line mode, no handshaking, exit

lda #$02

bit $dd01	; RS232 I/O port

bpl $f00d	; no DRS, error

bne $f012

.eff2  lda $02a1	; ENABL, RS232 enables

and #$02

bne $eff2

.eff9  bit $dd01	; RS232 I/O port

bvs $eff9	; wait for no CTS

lda $dd01

ora #$02

sta $dd01	; set RTS

.f006  bit $dd01

bvs $f012	; CTS set

bmi $f006	; wait for no DSR

F00D NO DSR ERROR

This routine sets the 6551 status register image to #40 when a no DSR error has occurred.

.f00d  lda #$40

sta $0297	; RSSTAT, 6551 status register image

.f012  clc

rts

F014 SEND TO RS232 BUFFER

Note: The entry point to the routine is at

if014  jsr if028

if017  ldy a029e

iny

cpy a029d

beq if014

sty a029e

dey

lda a9e

sta (pf9),y

if028  lda a02a1

lsr a

bcs if04c

lda #$10

sta add0e

lda a0299

sta add04

lda a029a

sta add05

lda #$81

jsr eef3b

jsr eef06

lda #$11

sta add0e

if04c  rts

F04D INPUT FROM RS232

if04d  sta a99

lda a0294

lsr a

bcc if07d

and #$08

beq if07d

lda #$02

bit add01

bpl if00d

beq if084

if062  lda a02a1

lsr a

bcs if062

lda add01

and #$fd

sta add01

if070  lda add01

and #$04

beq if070

if077  lda #$90

clc

jmp eef3b

if07d  lda a02a1

and #$12

beq if077

if084  clc

rts

F086 GET FROM RS232

if086  lda a0297

ldy a029c

cpy a029b

beq if09c

and #$f7

sta a0297

lda (pf7),y

inc a029c

rts

if09c  ora #$08

sta a0297

lda #$00

rts

F0A4 SERIAL BUS IDLE

This routine checks the RS232 bus for data transmission/reception. The routine waits for any activity on the bus to end before setting I.C.R. The routine is called by serial bus routines, since these devices use IRQ generated timing, and conflicts may occur if they are all used at once.

.f0a4  pha	; store (A)

lda $02a1	; ENABL, RS232 enables

beq $f0bb	; bus not in use

.f0aa  lda $02a1	; ENABL

and #$03	; test RS232

bne $f0aa	; yes, wait for port to clear

lda #$10

sta $dd0d	; set up CIA#2 I.C.R

lda #$00	; clear

sta $02a1	; ENABL

.f0bb  pla	; retrieve (A)

rts

F0BD TABLE OF KERNAL I/O MESSAGES 1

This is a table of messages used by the KERNAL in conjunction with its I/O routines. Bit 7 is set in the last character in each message as a terminator.

The table is split into two parts in the JiffyDOS kernal, since the tape messages have been removed, and being substituted by new routines.

.f0bd  0d 49 2f 4f 20 45 52 52 4f 52 20 a3	; i/o error #

.f0c9  0d 53 45 41 52 43 48 49 4e 47 a0	; searching

.f0d4  46 4f 52 a0		; for

F0D8 JIFFYDOS CLEAR SPRITES

This routine is called by JiffyDOS before executing timecritical routines that might be messed up by sprites on the screen. A loop is performed afterwards that lets sprites currently being displayd on the screen, to be finished.

.f0d8  lda #$00

sta $d015	; clear sprites

.f0dd  adc #$01	; perform loop

bne $f0dd	; 256 times

rts

F0E2 JIFFYDOS SET CHKIN

This routine is a new JiffyDOS routine which clears all I/O and sets up the current JiffyDOS filenumber as default inputdevice by calling CHKIN.

.f0e2  lda $9f	; JiffyDOS Logical Filenumber

.f0e4  pha	; store (A)

jsr $ffcc	; CLRCHN

pla	; retrieve (A)

tax	; (A) to (X)

jmp $ffc6	; CHKIN, open channel for input

F0ED JIFFYDOS SERIAL OUTPUT 1

This is a patch to the original Commodore KERNAL, that clears the flag that indicates a JiffyDOS device, ($a3), before setting the serial output to 1.

.f0ed  lda #$00	; clear JiffyDOS device flag

sta $a3

jmp $ee97	; serial output 1

F0F4 JIFFYDOS SEND DRIVE COMMAND

This routine uses the values in (X) and (Y) to send a command to the drive. (X) contains a offset to the command, and (Y) contains the length of the command.

.f0f4  txa	; temp store (X)

pha

jsr $f7a2	; open command channel for output

pla

tax	; retrieve (X)

.f0fb  lda $f398,x	; read command from table

jsr $ffd2	; output character to drive

inx	; next character

dey	; decrement counter

bne $f0fb	; till ready

rts

F106 TABLE OF KERNAL I/O MESSAGES 2

This is the second part of the KERNAL I/O message table. Part 1 is to be found at address $f0bd.

.f106  0d 4c 4f 41 44 49 4e c7	; loading

.f10e  0d 53 41 56 49 4e 47 a0	; saving

.f116  0d 56 45 52 49 46 59 49 4e c7	; verifying

.f120  0d 46 4f 55 4e 44 a0	; found

.f127  0d 4f 4b 9d		; ok

F12B PRINT MESSAGE IF DIRECT

This is a routine to output a message from the I/O messages table at $f0bd. On entry, (Y) holds the offset to control which message is printed. The routine tests if we are in program mode or direct mode. If in program mode, the routine exits. Else, the routine prints character after caracter untill it reaches a character with bit7 set.

.f12b  bit $9d	; MSGFLG, test if direct or program mode

bpl $f13c	; program mode, don't print message

.f12f  lda $f0bd,y	; get output character from table

php	; store processor registers

and #$7f	; clear bit7

jsr $ffd2	; output character using CHROUT

iny	; increment pointer to next character

plp	; retrieve message

bpl $f12f	; untill bit7 was set

.f13c  clc	; clear carry to indicate no error

rts

F13E GETIN: GET a BYTE

The KERNAL routine GETIN ($ffe4) is vectored to this routine. It load a character into fac#1 from the external device indicated by DFLTN. Thus, if device = 0, GET is from the keyboard buffer. If device = 2, GET is from the RS232 port. If niether of these devices then GET is further handled by the next routine, INPUT.

.f13e  lda $99	; DFLTN, default input device.

bne $f14a	; not keyboard

lda $c6	; NDX, number of keys in keyboard queue

beq $f155	; buffer empty, exit

sei	; disable interrupts

jmp $e5b4        ; get character from keyboard buffer, and exit

.f14a  cmp #$02	; RS232

bne $f166	; nope, try next device

.f14e  sty $97	; temp store

jsr $f086	; get character from RS232

ldy $97	; retrieve (Y)

.f155  clc

rts

F157 CHRIN: INPUT A BYTE

The KERNAL routine CHRIN ($ffcf) is vectored to this routine. It is similar in function to the GET routine above, and also provides a continuation to that routine. If the input device is 0 or 3, ie. keyboard or screen, then input takes place from the screen. INPUT/GET from other devices are performed by calls to the next routine. Two bytes are input from the device so that end of file can be set if necessary (ie. ST = #40)

.f157  lda $99	; DFLTN, default input

bne $f1a9	; not keyboard, next device

lda $d3	; PNTR, cursor column on  screen

sta $ca	; >LXSP, cursor position at start

lda $d6	; TBLX, cursor line number

sta $c9	; <LXSP

jmp $e632	; input from screen or keyboard

.f166  cmp #$03	; screen

bne $f173	; nope, next device

sta $d0	; CRSW, flag INPUT/GET from keyboard

lda $d5	; LNMX, physical screen line length

sta $c8	; INDX, end of logical line for input

jmp $e632	; input from screen of keyboard

.f173  bcs $f1ad

cmp #$02	; RS232

beq $f1b8	; yes, get data from RS232 port

.f179  jsr $fbaa	; JiffyDOS ACPTR, get byte from serial bus

pha	; temp store on stack

bit $a3	; test bit6, if serial device is a JiffyDOS device

bvc $f19c	; no JiffyDOS device

cpx #$00

bne $f187

lda $c4	; ??????

.f187  cmp #$04

bcc $f19c

ldy #$00	; clear offset

lda ($bb),y	; FNADR, pointer to current filename

cmp #$24	; first character is $, ie. directory

beq $f19c	; yes, exit

inc $b9	; increment SA

jsr $f38b	; execute TALK, and TKSA

dec $b9	; decrement SA

asl $a3

.f19c  pla

rts

.f19e  lda #$10	; set bit4

jmp $fe1c	; write to STATUS

F1A3 VECTOR TABLE

The following table contains three vectors that is copied to $0300 when the @X command is executed.

.f1a3  eb e3	; IERROR vector

.f1a5  83 a4	; IMAIN vector

.f1a7  7c a5	; ICRNCH vector

lda

sed a90

if1ad  =*+01

lda f90a5,y

F1AD GET FROM SERIAL/RS232

These routines, actually two different, is entered from the previous routine. The serial sectionchecks the state of ST. If zero, then the data is recieved from the bus, otherwise carriage return (#0d) is returned in (A). In the second section, the recieved byte is read from the RS232 port.

.f1ad  lda $90	; STATUS, I/O status word

beq $f1b5	; status OK

.f1b1  lda #$0d	; else return <CR> and exit

.f1b3  clc

.f1b4  rts

.f1b5  jmp $fbaa	; JiffyDOS ACPTR, get byte from serial bus

.f1b8  jsr $f14e	; receive from RS232

bcs $f1b4	; end with carry set

cmp #$00

bne $f1b3	; end with  carry clear

lda $0297	; RSSTAT, 6551 status register

and #$6	; mask

bne $f1b1	; return with <CR>

beq $f1b8	; get from RS232

F1CA CHROUT: OUTPUT ONE CHARACTER

The KERNAL routine CHROUT ($ffd2) is vectored to this routine. On entry, (A) must hold the character to be output. The default output device number is examined, and output directed to relevant device. The screen, serial bus and RS232 all use previously described routines for their output.

Some old taperoutines have been removed in the middle of this routine, and been changed to a JiffyDOS routine.

.f1ca  pha	; temp store on stack

lda $9a	; DFLTO, default output device

cmp #$03	; screen?

bne $f1d5	; nope, test next device

pla	; retrieve (A)

jmp $e716	; output to screen

.f1d5  bcc $f1db	; device <3

pla	; retrieve (A)

jmp $eddd	; send serial deferred

.f1db  lsr a

pla

sta $9e	; PTR1, some tape junk left in the code

txa

pha

tya

pha

bcc $f208	; RS232

jmp $f3f1	; output device not present

.f1e8  jsr $f8bf

jsr $e4c6

cmp #$30

rts

F1F1 JIFFYDOS DEFAULT DEVICE

The following routine sets the default device number. It uses the GTBYTC procedure to read the specifyed device number.

.f1f1  jsr $b79b	; GTBYTC, read device number from keyboardbuffer

stx $ba	; store in FA, current device number

jsr $f75c	; test if device FA is present.

stx $be	; If OK, store

rts

F1FC CHROUT: PART 2

This is the second part of the CHROUT routine. It contains the last parts of the RS232 output routine.

.f1fc  clc

pla

tay

pla

tax

lda $9e	; PTR1

bcc $f207

lda #$00

.f207  rts

.f208  jsr $f017	; send to RS232

jmp $f1fc	; end output

F20E CHKIN: SET INPUT DEVICE

The KERNAL routine CHKIN ($ffc6) is vectored to this routine. On entry, (X) must hold the logical file number. A test is made to see if the file is open, or ?FILE NOT OPEN. If the file is not an input file then ?NOT INPUT FILE. If the device is on the serial bus then it is commanded to TALK  and secondary address is sent. ST is then checked, and if non-zero, ?DEVICE NOT PRESENT. Finally, the device number is stored in DLFTN.

.f20e  jsr $f30f	; find file number

beq $f216	; ok, skip next command

jmp $f701	; I/O error #3, file not open

.f216  jsr $f31f	; set file variables

lda $ba	; FA, current device number

beq $f233	; keyboard

cmp #$03	; screen

beq $f233	; yes

bcs $f237	; larger than 3, serial bus device

cmp #$02	; RS232

bne $f22a	; nope

jmp $f04d	; input from RS232

.f22a  ldx $b9	; SA, current secondart address

cpx #$60

beq $f233

jmp $f70a	; I/O error #6, not output file

.f233  sta $99	; DFLTN, default input device

clc

rts

.f237  tax

jsr $ed09	; send TALK to serial device

lda $b9	; SA

bpl $f245	; send SA

jsr $edcc	; wait for clock

jmp $f248

.f245  jsr $edc7	; send talk secondary address

.f248  txa

bit $90	; STATUS, I/O status word

bpl $f233	; store DFLTN, and exit

jmp $f707	; I/O error #5, device not present

F250 CHKOUT: SET OUTPUT DEVICE

The KERNAL routine CHKOUT ($ffc9) is vectored to this routinr. On entry (X) must hold the logical filenumber. A test is made to see if the file is open, or ?FILE NOT OPEN error. If the device is 0, ie. the keyboard, or the file is not an output file, then ?FILE OUTPUT FILE error is generated. If the device is on the serial bus, then it commanded to LISTEN and the secondary address is sent. ST is then checked and if non-zero, then ?DEVICE NOT PRESENT error. Finally, the device number is stored in DFLTO.

.f350  jsr $f30f	; fine file number (X)

beq $f258	; OK

jmp $f701	; I/O error #3, file not open

.f258  jsr $f31f	; set file values

lda $ba 	; FA, current device number

bne $f262	; not keyboard

.f25f  jmp $f70d	; I/O error #7, not output file

.f262  cmp #$03	; screen?

beq $f275	; yes

bcs $f279	; serial bus device

cmp #$02	; RS232

bne $f26f	; nope

jmp $efe1	; submit to RS232

.f26f  ldx $b9	; SA, current secondary address

cpx #$60

beq $f25f	; not output file error

.f275  sta $9a	; DFLTO, default output device

clc	; clear carry to incicate no errors

rts

.f279  tax	; file (X) to (A)

jsr $ed0c	; send LISTEN to serial device

lda $b9	; SA

bpl $f286        ; send SA

jsr $edbe	; clear ATN

bne $f289

.f286  jsr $edb9	; send listen secondary address

.f289  txa

bit $90	; STATUS, I/O status word

bpl $f275	; OK, set output device

jmp $f707 	; I/O error #5, device not present

F291 CLOSE: CLOSE FILE, PART 1

The KERNAL routine CLOSE ($ff3c) is vectored here. The file parameters are fetched, and if not found, the routine exits without any action. It checks the device number associated with the file. If it is RS232, then the RS232 port is reset. If it is a serial device, the device is UNTALKed, or UNLISTENed. Finally the number of open logical files are decremented, and the table of active file numbers are updated. On entry (A) holds the file number to close. Old tape routines ($f2cc-$f2e1) has been removed for new JiffyDOS routines.

.f291  jsr $f314	; find logical file, (X) holds location i table

beq $f298	; OK

clc	; file not found

rts	; and exit

.f298  jsr $f31f	; get file values from table, position (X)

txa

pha	; temp store

lda $ba	; FA, currend device number

beq $f2f1	; keyboard?, update file table

cmp #$03	; screen

beq $f2f1	; yepp, update file table

bcs $f2ee	; Serial bus

cmp #$02	; RS232

bne $f2c8	; nope, serial

pla	; retriev (A)

jsr $f2f2	; remove entry (A) from file table

jsr $f483	; init RS232 port by using part of RS232OPEN

jsr $fe27	; MEMTOP, read top of memory (X/Y)

lda $f8	; >RIBUF, RS232 input buffer

beq $f2ba

iny

.f2ba  lda $fa	; >ROBUF, RS232 output buffer

beq $f2bf

iny

.f2bf  lda #$0	; Clear RS232 input/output buffers

sta $f8

sta $fa

jmp $f47d	; Set new ROBOF values and set new MEMTOP

.f2c8  pla      	; retriev (A)

jmp $f713

.f2cc  jsr $ffcc	; CLRCHN, close all channels

.f2cf  lda #$6f

jsr $f314	; FIND FILE, test if file number #$6f is open.

bne $f30e	; file not open, return

jmp $f2f3	; close file #$6f

F2D9 JIFFYDOS TEST DEVICE

The following routine tests if a device is present. On entry (X) holds the device to be tested. Open to the device is performed, and afterwards the statusword can be read for result.

.f2d9  stx $ba	; store (X) in FA

.f2db  tya

pha

jsr $f8b2	; open 15,x,15

jsr $f7a2	; set command channel (15) as output

php

jsr $f2cc	; close command channel

plp

pla

tay

ldx $ba

rts

sed $

F2e CLOSE: CLOSE FILE, PART 2

.f2ee  jsr $f642	; UNTALK/UNLISTEN serial device

.f2f1  pla

.f2f2  tax

.f2f3  dec $98	; decrement LDTND, number of open files

cpx $98	; compare LDTND to (X)

beq $f30d	; equal, closed file = last file in table

ldy $98	; else, move last entry to position of closed entry

lda $0259,y	; LAT, active filenumbers

sta $0259,x

lda $0263,y	; FAT, active device numbers

sta $0263,x

lda $026d,y	; SAT, active secondary addresses

sta $026d,x

.f30d  clc

.f30e  rts	; return

F30F FIND FILE

This routine finds a logical file from it's file number. On entry, (X) must hold the logical file number to be found. LAT, the table of file numbers is searched, and if found (X) contains the offset to the position of the file in the table, and the Z flag is set. If not found, Z=0.

.f30f  lda #$00

sta $90	; clear STATUS

txa	; file number to search for

.f314  ldx $98	; LDTND, number of open files

.f316  dex

bmi $f32e	; end of table, return

cmp $0259,x	; compare file number with LAT, table of open files

bne $f316	; not equal, try next

rts	; back with Z flag set

F31F SEET FILE VALUES

This routine sets the current logical file number, device number and secondary address from the file parameter tables. On entry (X) must hold the offset to the position of the file in the table.

.f31f  lda $0259,x      ; LAT, table of active logical files

sta $b8	; store in LA

lda $0263,x	; FAT, table of active device numbers

sta $ba	; store in FA

lda $026d,x	; SAT, table of active secondary addresses

sta $b9	; store in SAT

.f32e  rts	; return

F32F CLALL: ABORT ALL FILES

The KERNAL routine CLALL ($ffe7) is vectored here. The number of open files are set to zero, and the next routine is performed.

.f32f  lda #$00

sta $98	; clear LDTND, no open files

F333 CLRCHN: RESTORE TO DEFAULT I/O

The KERNAL routine CLRCHN ($ffcc) is vectored here. The default output device is UNLISTENed, if it is on the serial bus, and the default output is set to the screen. The default input device is UNTALKed, if it is on the serial bus, and the default input device is set to keyboard.

.f333  ldx #$03	; check if device > 3 (serial bus is 4,5...)

cpx $9a	; test DFLTO, default output device

bcs $f33c	; nope, no serial device

jsr $edfe	; send UNLISTEN to serial bus

.f33c  cpx $99	; test DFLTI, default input device

bcs $f343	; nope, no serial device

jsr $edef        ; send UNTALK to serial bus

.f343  stx $9a	; store screen as DFLTO

lda #$00

sta $99	; store keyboard as DFLTI

rts

F34A OPEN: OPEN FILE

The KERNAL routine OPEN ($ffc0) is vectored here. The file paramerters must be set before entry. The routine reads the LAT, to see if file already exists, which will result in I/O error #2, ?FILE OPEN. A test is made to see if more than 10 files are open. If so, I/O error #1, ?TOO MANY FiLES, will occur. The file parameters are set, and put in their respective tables. The device number is checked, and each kind of device jumps to their own routine. Keyboard and screen will exit here with no further actions. RS232 is opened via a seperate routine. SA, secondary address, and filename will be sent on the serial bus. Some tape routines are removed, and replaced with JiffyDOS code.

.f34a  ldx $b8	; LA, current logical number

bne $f351

jmp $f70a	; I/O error #6, not input file

.f351  jsr $f30f	; find file (X)

bne $f359

jmp $f6fe	; I/O error #2, file exists

.f359  ldx $98	; LDTND, number of open files

cpx #$0a	; more than ten

bcc $f362	; nope

jmp $f6fb	; I/O error #1, too many files

.f362  inc $98	; increment LDTND

lda $b8	; LA

sta $0259,x	; store in LAT, table of active file numbers

lda $b9	; SA

ora #$60	; fixx

sta $b9	; store in SA

sta $026d,x	; store in SAT, table of active secondary addresses

lda $ba	; FA

sta $0263,x	; store in FAT, table of active device numbers

beq $f3d3	; keyboard, end

cmp #$03	; screen

beq $f3d3	; yep, end

bcc $f384	; less than 3, not serial bus

jsr $f3d5	; send SA

bcc $f3d3	; end

.f384  cmp #$01	; TAPE

beq $f3f3	; I/O error #5, device not present

jmp $f409	; open RS232 file

F38B JIFFYDOS TALK & TKSA

This is a routine used by JiffyDOS to untalk device (A), then TALK and TKSA is executed to current device with current secondary address.

.f38b  jsr $ffab	; UNTALK

lda $ba	; FA, current device number

jsr $ffb4	; TALK

lda $b9	; SA, current secondary address

jmp $ff96	; TKSA, send SA after TALK

F398 JIFFYDOS DIRECT DRIVE COMMANDS

The following text/code is used to transfer, and is transfered to a selected drive. The first section is a $22 byte long block used by the lock/unlock a file. The second section is code to execute a drive program at $0600. The third section sets a byte in the drive memory to control the interleave. The fourth section sets a byte in the drive memory to control the 1541 head rattle.

.f398  4d 2d 57 00 06 1c  ; M-W 00 06 1c, ie. write $1c bytes to $0600

lda $0261          ; the following code is transfered to the drive

sta $07            ; at $0600

lda #$12

sta $06

ldx #$00

stx $f9

jsr $d586

ldy $0267

lda ($30),y

eor #$40

sta ($30),y

jmp $d58a

.f3b0  4d 2d 45 00 06     ; M-E 00 06, ie. a memory execute at $0600

.f3b6  4d 2d 57 6a 00 01  ; M-W 6a 00 01, ie. memory write one byte at $006a

.f3bc  4d 2d 57 69 00 01  ; M-W 69 00 01, ie. memory write one byte at $0069

ora (p50,x)

ror a01

sed $

rol $

sta aa6

if3d3  clc

rts

F3D5 SEND SA

This routine exits if there is no secondary address or filename specifyed. The I/O status word, ST, is reset, and the serial device is commanded to LISTEN. A check is made for a possible ?DEVICE NOT PRESENT error. Finally, the filename is sent to the device.

.f3d5  lda $b9	; SA, current secondary address

bmi $f3d3	; exit

ldy $b7	; FNLEN, length of filename

beq $f3d3	; exit

lda #$00

sta $90	; clear STATUS, I/O status word

lda $ba	; FA, current device number

jsr $ed0c	; send LISTEN to serial bus

lda $b9	; SA

ora #$f0

jsr $edb9	; send LISTEN SA

lda $90	; STATUS

bpl $f3f6	; ok

.f3f1  pla	; remove two stack entries for RTS command

pla

.f3f3  jmp $f707	; I/O error #5, device not present

.f3f6  lda $b7	; FNLEN

beq $f406	; unlisten and exit

ldy #$00	; clear offset

.f3fc  lda ($bb),y	; FNADR, pointer to filename

jsr $eddd	; send byte on serial bus

iny	; next character

cpy $b7	; until entire filename is sent

bne $f3fc	; again

.f406  jmp $f654	; unlisten and exit

F409 OPEN RS232

.f409  jsr $f483

sty a0297

.f40f  cpy ab7

beq if41d

lda (pbb),y

sta f0293,y

iny

cpy #$04

bne if40f

if41d  jsr eef4a

stx a0298

lda a0293

and #$0f

beq if446

asl a

tax

lda a02a6

bne if43a

ldy ffec1,x

lda ffec0,x

jmp if440

if43a  ldy fe4eb,x

lda fe4ea,x

if440  sty a0296

sta a0295

if446  lda a0295

asl a

jsr eff2e

lda a0294

lsr a

bcc if45c

lda add01

asl a

bcs if45c

jsr if00d

if45c  lda a029b

sta a029c

lda a029e

sta a029d

jsr efe27

lda af8

bne if474

dey

sty af8

stx af7

if474  lda afa

bne if47d

dey

sty afa

stx af9

if47d  sec

lda #$f0

jmp efe2d

if483  lda #$7f

sta add0d

lda #$06

sta add03

sta add01

lda #$04

ora add00

sta add00

ldy #$00

sty a02a1

rts

F49E LOAD: LOAD RAM

The kernal routine LOAD ($ffd5) is vectoed here. If a relocated load is desired, then the start address is set in MEMUSS. The load/verify flag is set, and the I/O status word is reset. A test is done on the device number, less than 3 results in illigal device number.

.f49e  stx $c3	; MEMUSS, relocated load address

sty $c4

jmp ($0330)	; ILOAD vector. Points to $f4a5

.f4a5  sta $93	; VRECK, load/verify flag

lda #$00

sta $90	; clear STATUS, I/O status

lda $ba	; get FA, current device

bne $f4b2	; keyboard

.f4af  jmp $f713	; I/O error #9, illigal device

.f4b2  cmp #$03	; screen?

beq $f4af	; yes, illigal device

F4B8 LOAD FROM SERIAL BUS

A filename is assumed by the routine, and if not present, a jump is made to a new JiffyDSO routine that sets filename to ':*'. The message 'SEARCHING' is printed and the filename is sent with the TALK command and secondary address to the serial bus. If EOI occurs at this point, then ?FILE NOT FOUND is displayed. The message 'LOADING' or 'VERIFYING' is output and a loop is entered, which recieves a byte from the serial bus, checks the <STOP> key and either stores the received byte, or compares it to the memory, depending on the state of VERCK. Finally the bus is UNTALKed.

.f4b8  bcc $f4af	; device < 3, eg tape or RS232, illigal device

ldy $b7	; FNLEN, length of filename

bne $f4bf	; if length not is zero

jmp $f659	; fixx filename, JiffyDOS patch

.f4bf  ldx $b9	; SA, current secondary address

jsr $f5af	; print "SEARCHING"

lda #$60

sta $b9	; set SA to $60

jsr $f3d5	; send SA and filename

lda $ba	; FA, current devicenumber

jsr $ed09	; send TALK to serial bus

lda $b9	; SA

jsr $edc7	; send TALK SA

jsr $ee13	; receive from serial bus

sta $ae	; load address, <EAL

lda $90	; check STATUS

lsr a

lsr a

bcs $f530	; EOI set, file not found

jsr $f179	; recieve from serial bus

sta $af	; load address, >EAL

txa	; retrieve SA and test relocated load

bne $f4f0	;

lda $c3	; use MEMUSS as load address

sta $ae	; store in <EAL

lda $c4

sta $af	; store in >EAL

.f4f0  jmp $fac4	; jump to JiffyDOS patch

.f4f3  jsr $ffe1	; scan <STOP>

bne $f4fb	; not stopped

jmp $f633

.f4fb  jsr $fbaa	; JiffyDOS ACPTR, recrive from serial bus

lda $90	; read ST

and #$fd	; mask %11111101

cmp $90

sta $90

bne $f4f3	; EOI set

ldy #$00

ldx $a3

lda $a4	;

cpy $93	; VERIFY eller LOAD

beq $f51a	; jump to LOAD

cmp ($ae),y	; compare with memory

beq $f51c	; veryfied byte OK

jsr $f19e	;

.byte $2c	; mask next write command

.f51a  sta ($ae),y	; store in memory

.f51c  stx $a3

inc $ae	; increment <EAL, next address

bne $f524	; skip MSB

inc $af	; increment >EAL

.f524  bit $90	; test STATUS

bvc $f4f3	; get next byte

jsr $edef	; send UNTALK to serial bus

jsr $f642

bcc $f5a9	; end routine

.f530  jmp $f704	; I/O error #4, file not found

F533 JIFFYDOS @ COMMAND

The following routine executes the @ command. First it tests if additional parameters are entered.

.f533  lda $b7	; FNLEN, length of current filename

beq $f546	; no filename

lda ($bb),y	; test filename for

cmp #$24	; $, directory

beq $f56c	;

jmp $fc9a	; else goto

F540 JIFFYDOS LIST ASCII FROM DISK

This routine lists an ascii file from disk. It reads one block of text from the disk (254 bytes) into the filename area. The text is then output using the 'print filename' routine.

.f540  tya	; (Y) contains the command number

pha	; store on stack

jsr $f8bf	; open file with current parameters

pla	; retrieve

.f546  sta $a6 	; store

.f548  jsr $f911	; input charaters to buffer (filename area)

bne $f568	; exit if errors occured

lda $a6	; get command number, should be $0f

php

beq $f557

jsr $e4c6	; input byte from command channel

beq $f567	; if byte =# then exit

.f557  jsr $f79a

jsr $f5c1	; print filename, ie. the input buffer

bit $91	; STKEY FLAG, test if <STOP> is pressed

bpl $f567	; exit

plp

bne $f548

bvc $f548

.byte $24	; mask one byte, ie. PLP command

.f567  plp

.f568  rts

F569 JIFFYDOS BASIC DISC LIST

The following routine reads the specifyed basic-file from disk and displays it to the screen. The entrypoint at $f56c is used for showing the directory. First, the routine opens the file specifyed. IERROR vector is changed to $f739, so a RTS command will be performed when a error occurs. Then the start address is read, and thrown away. A loop is performed that reads one block of bytes from the disk and is output through the basic LIST routine. On exit, the IERROR vector is restored.

.f569  ldx #$6c	; get byte for SA, list basic program

.byte $2c	; mask next 2 bytes

.f56c  ldx #$60	; get byte for SA, list directory

jsr $f8c1	; open file with current parameters

lda #$39	; setup JiffyDOS IERROR vector to point to

sta $0300        ; $f739, a RTS-command

ldy #$fc	; set up (Y) pointer to 252

jsr $fca6	; read two garbage bytes (program start address)

.f57b  ldy #$00	; set up (Y) pointer to 0

.f57d  jsr $fca6	; read 254 bytes, store in input buffer

bvs $f5a3	; EOI, exit

cpy #$02	; if (Y) = 2

beq $f5a3	; exit

cpy #$06

bcc $f57d

ldx $bb	; read FNADR pointer, vector to input buffer

stx $5f	; store in temp vector

ldx $bc

stx $60

ldy #$01

sta ($5f),y

jsr $a6c3	; use part of LIST routine to output text

jsr $f79a

jsr $a6d4

bit $91	; STKEY FLAG, stop key

bmi $f57b	; not pressed, continue

.f5a3  lda #$63	; restore JiffyDOS IERROR vector to $f763

sta $0300	; IERROR VEC

rts

F5A9 LOAD END

This is the last part of the loader routine which sets the (X/Y) register with the endaddress of the loaded program, clears carry and exit.

.f5a9  clc

ldx $ae

ldy $af

rts

F5AF PRINT "SEARCHING"

If MSGFLG indicates program mode then the message is not printed, otherwise the message "SEARCHING" is printed from the KERNAL I/O message table. If the length of filename > 0 then the message "FOR" is printed, and the routine drops through to print the filename.

.f5af  lda $9d	; MSGFLG, direct or program mode?

bpl $f5d1	; program mode, don´t print, exit

ldy #$0c

jsr $f12f	; print "SEARCHING"

lda $b7	; FNLEN, length of current filename

beq $f5d1	; no name, exit

ldy #$17

jsr $f12f	; print "FOR"

F5C1 PRINT FILENAME

Filename is pointed to by FNADR, and length in FNLEN. The KERNAL routine CHROUT is used to print filename.

.f5c1  ldy $b7	; FNLEN, length of current filename

beq $f5d1	; exit

ldy #$00

.f5c7  lda ($bb),y	; get character in filename

jsr $ffd2	; output

iny	; next character

cpy $b7	; ready?

bne $f5c7

.f5d1  rts	; back

F5D2 PRINT "LOADING/VERIFYING"

The load/verify flag is checked, and if the message to be output is flagged according to the result. This message is printed from the KERNAL I/O messages table.

.f5d2  ldy #$49	; offset to verify message

lda $93	; VERCK, load/verify flag

beq $f5da	; verify

ldy #$59	; offset to load message

.f5da  jmp $f12b	; output message flagged by (Y)

F5DD SAVE: SAVE RAM

The KERNAL routine SAVE ($ffd8) jumps to this routine. On entry, (X/Y) must hold the end address+1 of the area of memory to be saved. (A) holds the pointer to the start address of the block, held in zeropage. The current device number is checked to ensure that it is niether keyboard (0) or screen (3). Both of these result in ?ILLIGAL DEVICE NUMBER.

.f5dd  stx $ae	; EAL , end address of block +1

sty $af

tax	; move start pointer to (X)

lda $00,x

sta $c1	; STAL, start address of block

lda $01,x

sta $c2

jmp ($0332)	; vector ISAVE, points to $f5ed

.f5ed  lda $ba	; FA, current device number

bne $f5f4	; ok

.f5f1  jmp $f713	; I/O error #9, illigal device number

.f5f4  cmp #$03	; screen?

beq $f5f1	; yep, output error

bcc $f5f1	; less than 3, ie. tape, output error

F5FA SAVE TO SERIAL BUS

A filename is assumed by the routine, or ?MISSING FILENAME error is called. The serial device is commanded to LISTEN, and the filename is sent along with the secondary address. The message 'SAVING' is printed, and a loop sends a byte to the serial bus and checks <STOP> key until the whole specifyed block of memory has been saved. Note that the first two bytes sent are the start address of the block. Finally the serial bus is UNLISTENed.

.f5fa  lda #$61

sta $b9	; set SA, secondary address, to #1

ldy $b7	; FNLEN, length of current filename

bne $f605	; ok

.f602  jmp $f710	; I/O error #8, missing filename

.f605  jsr $f3d5	; send SA & filename

jsr $f68f	; print 'SAVING' and filename

lda $ba	; FA, current device number

jsr $ed0c	; send LISTEN

lda $b9	; SA

jsr $edb9	; send LISTEN SA

ldy #$00

jsr $fb8e	; reset pointer

lda $ac	; SAL, holds start address

jsr $eddd	; send low byte of start address

lda $ad

jsr $eddd	; send high byte of start address

.f624  jsr $fcd1	; check read/write pointer

bcs $f63f

lda ($ac),y	; get character from memory

jsr $eddd	; send byte to serial device

jsr $ffe1	; test <STOP> key

bne $f63a	; not pressed

.f633  jsr $f642	; exit and unlisten

lda #$00	; flag break

sec

rts

.f63a  jsr $fcdb	; bump r/w pointer

bne $f624	; save next byte

.f63f  jsr $edfe	; send UNLISTEN

.f642  bit $b9	; SA

bmi $f657

lda $ba	; FA

jsr $ed0c	; send LISTEN

lda $b9

and #$ef

ora #$e0

jsr $edb9	; send UNLISTEN SA

.f654  jsr $edfe	; send UNLISTEN

.f657  clc

rts

F659 JIFFYDOS DEFAULT FILENAME

The following routine is executed when a missing filename is detected in the original loader routine. If so, the filename is set to ':*', wildcard filename. On exit, a jump is made to the original loader with new filename parameters set.

.f659  lda $c6	; NDX, number of characters in keyboard buffer

beq $f602	; if zero, output missing filename error

lda #$02	; store $02

sta $b9	; in SA, default secondary address

ldx #$74	; set up filename pointer to $f674

ldy #$f6	; ie. ':*'

jsr $ffbd	; SETNAM

jmp $f4bf	; back to loader routine

ldx #$33	; offset

ldy #$04	; length

jmp $f932	; drive command

F672 JIFFYDOS FUNKTION KEYS

The following table contains the strings copyed to the keyboard buffer when the funktionkeys are pressed. This table is pointed to by the FNKVEC at $b0/$b1. The strings are seperated by a zero-byte.

.f672  40 24 3a 2a 0d 00	; F1 = '@$:*'

.f678  2f 00		; F3 = '/'

.f67a  5e 00		; F5 = arrow up

.f67c  25 00		; F7 = '%'

.f67e  40 44 00		; F2 = '@d'

.f681  40 54 00		; F4 = '@t'

.f684  5f 00		; F6 = arrow left

.f686  40 20 20 22 53 3a 00	; F8 = '@  "S:'

clc

if68e  rts

F68F PRINT 'SAVING'

MSGFLG is checked, and if direct mode is on, then the message 'SAVING' is flagged and printed from the KERNAL I/O message table.

.f68f  lda $9d	; MSGFLG

bpl $f68e	; not in direct mode, exit

ldy #$51	; offset to message in table

jsr $f12f	; output 'SAVING'

jmp $f5c1	; output filename

F69B UDTIM: BUMP CLOCK

The KERNAL routine UDTIM ($ffea) jumps to this routine. The three byte jiffy clock in RAM is incremented. If it has reached $4f1a01, then it is reset to zero. this number represents 5184001 jiffies (each jiffy is 1/60 sec) or 24 hours. finally, the next routine is used to log the CIA key reading.

.f69b  ldx #$00

inc $a2	; low byte of jiffy clock

bne $f6a7

inc $a1	; mid byte of jiffy clock

bne $f6a7

inc $a0	; high byte of jiffy clock

.f6a7  sec

lda $a2	; substract $4f1a01

sbc #$01

lda $a1

sbc #$1a

lda $a0

sbc #$4f

bcc $f6bc	; and test carry if 24 hours

stx $a0	; yepp, reset jiffy clock

stx $a1

stx $a2

F6BC LOG CIA KEY READING

This routine tests the keyboard for either <STOP> or <RVS> pressed. If so, the keypress is stored in STKEY.

.f6bc  lda $dc01	; keyboard read register

cmp $dc01

bne $f6bc	; wait for value to settle

tax

bmi $f6da

ldx #$bd

stx $dc00	; keyboard write register

.f6cc  ldx $dc01	; keyboard read register

cpx $dc01

bne $f6cc	; wiat for value to settle

sta $dc00

inx

bne $f6dc

.f6da  sta $91	; STKEY, flag STOP/RVS

.f6dc  rts

F6DD RDTIM: GET TIME

The KERNAL routine RDTIM ($ffde) jumps to this routine. The three byte jiffy clock is read into (A/X/Y) in the format high/mid/low. The routine exits, setting the time to its existing value in the next routine. The clock resolution is 1/60 second. SEI is included since part of the IRQ routine is to update the clock.

.f6dd  sei	; disable interrupt

lda $a2	; read TIME

ldx $a1

ldy $a0

F6E4 SETTIM: SET TIME

The KERNAL routine SETTIM ($ffdb) jumps to this routine. On entry, (A/X/Y) must hold the value to be stored in the clock. The forman is high/mid/low, and clock resolution is 1/60 second. SEI is included since part of the IRQ routine is to update the clock.

.f6e4  sei	; disable interrupt

sta $a2	; wrine TIME

stx $a1

sty $a0

cli	; enable interrupts

rts

F6ED STOP: CHECK <STOP> KEY

The KERNAL routine STOP ($ffe1) is vectored here. If STKEY =#7f, then <STOP> was pressed and logged whilest the jiffy clock was being updated, so all I/O channels are closed and the keyboard buffer reset.

.f6ed  lda $91	; STKEY

cmp #$7f	; <STOP> ?

bne $f6fa	; nope

php

jsr $ffcc	; CLRCHN, close all I/O channels

sta $c6	; NDX, number of characters in keyboard buffer

plp

.f6fa  rts

F6FB OUTPUT KERNAL ERROR MESSAGES

The error message to be output is flagged into (A) depending on the entry point. I/O channels are closed, and then if KERNAL messages are enabled, "I/O ERROR #" is printed along with the error number.

.f6fb  lda #$01	; error #1, too many files

.byte $2c

.f6fe  lda #$02	; error #2, file open

.byte $2c

.f701  lda #$03	; error #3, file not open

.byte $2c

.f704  lda #$04	; error #4, file not found

.byte $2c

.f707  lda #$05	; error #5, device not found

.byte $2c

.f70a  lda #$06	; error #6, not input file

.byte $2c

.f70d  lda #$07	; error #7, not output file

.byte $2c

.f710  lda #$08	; error #8, missing filename

.byte $2c

.f713  lda #$09	; error #9, illigal device number

pha

jsr $ffcc	; CLRCHN, close all I/O channels

ldy #$00

bit $9d	; test MSGFLAG, KERNAL messages enabled

bvc $f729	; no

jsr $f12f	; print "I/O ERROR #"

pla

pha

ora #$30	; convert (A) to ASCII number

jsr $ffd2	; use CHROUT to print number in (A)

.f729  pla

sec

rts

F72C TEST JIFFY COMMAND

This routine test the character in the current key in the buffer if it is a JiffyDOS command character. Output from this routine is (Y) which contains the value of the selected command. (Y)=$ff if no command was found.

.f72c  ldy #$0c	; number of characters to test

jsr $79	; CHARGOT, read current character in buffer again

.f731  cmp $f7dd,y	; equal to byte in JiffyDOS command tab

beq $f739	; yepp, return

dey	; test next

bpl $f731	; till (Y)=$ff

.f739  rts	; back

F73A JIFFYDOS SLPARA

This routine is executed from the original SLPARA. It executes SETLFS to set logical file parameters, as normal. But it also continues through the next routine to find a present device number.

.f73a  jsr $ffba	; SETLFS

.F73D JIFFYDOS TEST SERIAL DEVICE

This routine tests a serial disk device number to see if it is present. The routine uses $be as a internal counter for device number. A test is performed to make sure that the device number is within its limits, $08-$1f. If a device is not present, the routine continues searching for a present device. The second time we reset the counter to $08 (after reaching $1f) without finding a device, the routine exits with error #5, device not present.

.f73d  clc	; clear carry

php	; store carry

ldx $be	; internal counter for devicenumber

cpx #$08	; device $8

bcc $f749	; less than $8

.f745  cpx #$1f	; serial device must be less than $1f (31)

bcc $f750	; less than $1f

.f749  plp	; if carry set, this is second time

bcs $f761	; do error

sec	; set carry to indicate first reset

php	; store carry

ldx #$08	; start at $08 again

.f750  stx $be	; store

jsr $f2d9	; test devicenummer (X)

bcc $f75a	; OK, device (X) is next present device

inx	; next devicenumber

bne $f745	; test next

.f75a  pla	; clean ut stack

.f75b  rts	; exit

.f75c  jsr $f2db	; test devicenumber in FA

bcc $f75b	; ok

.f761  ldx #$05	; ERROR, device not present

F763 IERROR: JIFFYDOS ERROR ROUTINE

The ERROR vector IERROR ($0300) points to this routine. On entry (X) holds the error number. A test is done to see if this is a SYNTAX error ($0b). If not, it jumps to the original IERROR at $e38b, where errors are taken care of as usual. Else, the routine continues by checking if the error was caused by a JiffyDOS command.

.f763  cpx #$0b	; SYNTAX ERROR

beq $f76a	; yes, jump to command test

.f767  jmp $e38b	; nope, normal error handler

F76A COMMAND: TEST FOR EXTRA JIFFYDOS COMMANDS

The following routine tests if a JiffyDOS command has been entered. A subroutine is called to test this, and it leavs the JiffyDOS command number in (Y), if any found. It tests for a present serial device,

.f76a  jsr $f72c	; test JiffyDOS command. On exit, (Y)=command number

bne $f767	; no JiffyDOS command

sty $27	; temp store

tax

bmi $f776

pla

pla

.f776  jsr $f73d	; test serial device, if any present

jsr $f838	; command after '@'? Setname and open specifyed file

lda $27	; retrieve temp, command number.

ldy #$00

asl a	; times 2

tax	; to (X)

lda $f7f5,x	; get low commandvector

sta $55	; store

lda $f7f6,x	; get high commandvector

sta $56	; store

.f78c  jsr $54	; execute JiffyDOS command

jsr $a8f8	; ignore next statement, sort of REM

jsr $f2cc	; close all channels, and file 15 if open

lda $9f	; JiffyDOS default filenumber

jsr $ffc3	; CLOSE

.f79a  jsr $ffcc	; CLRCHN, close all I/O channels

ldx $13	; CHANNL

beq $f75b	; screen and keyboard are current I/O device, exit

.byte $2c	; else mask next LDX-command, and perform CHKOUT

.f7a2  ldx #$6f	; command channel

jmp $ffc9	; CHKOUT, open channel for output.

F7A7 JIFFYDOS ML-LOAD

This is the entrypoint for £ and %, which loads machine language

.f7a7  tya

iny

.byte $2c	; bit $xxxx, trick to skip 2 commands

F7AA JIFFYDOS VERIFY

This is the entrypoint for ´,which veryfies a file

.f7aa  tya

F7AB JIFFYDOS BASIC-LOAD

This is the entrypoint for / and 'arrow up' which loads a basic program. The LOAD/VERIFY is performed. Depending on what command is executed, various end routines are performed.

.f7ab  iny

sty $b9	; SA, current secondary address

ldx $2b	; TXTTAB, start of basic

ldy $2c

jsr $ffd5	; LOAD

bcc $f7c0	; load OK

jmp $e0f9	; handle I/O error

.f7ba  jmp $e195	; load OK?

.f7bd  jmp $e17e	; verify OK?

.f7c0  lda $27	; test command number

cmp #$0b	; verify command (´)

beq $f7bd	; output verify OK

bcs $f78c	; command number larger than $0b

cmp #$08	; load ml (%)

beq $f75b	; if so exit

bcc $f7ba	; if command number less than 8, test if OK and exit

stx $2d	; VARTAB, set start af Basic variables

sty $2e

pla	; remove RTS return address

pla

jsr $aad7	; output CR/LF

jsr $a533	; rechain basic lines

jmp $a871	; perform RUN

F7DD JIFFYDOS COMMAND TAB

The tab contains the additional JiffyDOS commands. The $0c first commands can be entered at the prompt, and are tested at $f72c. The remaing commands must be entered after the @-character. The DOS 5.1 Wedge Commands are not checked here.

.f7dd  40	; @

.f7de  5f	; <-

.f7df  2a	; *

.f7e0  ac	; dot in lower right corner. (Same as *. Possible 	; future expansion)

.f7e1  22	; "

.f7e2  12	; . (Same as ". Possible future expansion)

.f7e3  2f	; /

.f7e4  ad	; right angle in top right corner. (Same as /. 		  Possible future expansion)

.f7e5  25	; %

.f7e6  5e	; arrow up

.f7e7  ae	; right angle in lower left corner. (Same as 'arrow 	  up'. Possible future expansion)

.f7e8  27	; ´

.f7e9  5c	; £

The following command characters must be entered after the @-character.

.f7ea  44	; D

.f7eb  4c	; L

.f7ec  54	; T

.f7ed  23	; #

.f7ee  42	; B

.f7ef  46	; F

.f7f0  4f	; O

.f7f1  50	; P

.f7f2  51	; Q

.f7f3  58	; X

.f7f4  47	; G

F7F5 JIFFYDOS COMMAND VECTORS

The following table contains the JiffyDOS command vectors. The vectors are in the same order as the command characters above.

.f7f5  33 f5	; execute @ at $f533

.f7f7  59 e1	; execute <- at $e159

.f7f9  39 fa	; execute * at $fa39

.f7fb  39 fa	; execute XX at $fa39

.f7fd  2b f7	; execute " at $f72b

.f7ff  2b f7	; execute . at $f72b

.f801  ab f7	; execute / at $f7ab

.f803  ab f7	; execute XX at $f7ab

.f805  a7 f7	; execute % at $f7a7

.f807  ab f7	; execute 'arrow up' at $f7ab

.f809  ab f7	; execute XX at $f7ab

.f80b  aa f7	; execute ´ at $f7aa

.f80d  a7 f7	; execute £ at $f7a7

The following commands are extra JiffyDOS commands and to come after the @-character

.f80f  69 f5	; execute D at $f569

.f811  d4 f8	; execute L at $f8d4

.f813  40 f5	; execute T at $f540

.f815  f1 f1	; execute # at $f1f1

.f817  2c f9 	; execute B at $f92c

.f819  c2 e4	; execute F at $e4c2

.f81b  25 f8	; execute O at $f825

.f81d  97 fa	; execute P at $fa97

.f81f  bc fc	; execute Q at $fc2c

.f821  a0 fc	; execute X at $fca0

.f823  24 f9	; execute G at $f924

F825 JIFFYDOS OLD

The following routine performs a basic old after a new or reset. The routine performs a rechain to set up correct pointers etc.

.f825  iny

tya

sta ($2b),y	; store XX in $08XX to reinit basic

jsr $a533	; LINKPRG, rechain basic lines

txa

adc #$02

tax

lda $23

adc #$00	; (X) and (Y) contains start of variables

tay

jmp $e1a7	; set start of variables, and restart basic.

F838 JIFFYDOS COMMAND PART 2

This routine is called from the JiffyDOS COMMAND routine and make a test for additional command characters after the '@' character. Only the command number $0d-$17 is tested. If text after '@' is not a JiffyDOS command (ie. a normal DOS command', or JiffyDOS command number less than $10, a filename is expected. Tests are made for colon and quotes, the filname is evaluated, and parts of the OPEN/CLOSE routine is used to SETNAM. A test is made for additional device number after a comma. A free line on the screen is found, and some string-house keeping is done. Finally, the routine continues through to the next routine to open the command channel.

.f838  tya

bne $f853

sta $b7

.f83d  jsr $73	; CHRGET, get character from buffer

beq $f887	; terminator found, exit

ldy #$17	; set pointer for start of command

jsr $f731	; test if character is JiffyDOS command

bne $f858	; nope, no command

cpy #$0d	; only test value $17 to $0d

bcc $f858	; if less than $0d, exit

sty $27	; temp store

cpy #$10	; read command value

bcs $f887	; if value larger than $10, filename is not expected

.f853  lda #$01

jsr $a8fc	; add TXTPTR by one

.f858  ldy #$ff	; init pointer

.f85a  iny

lda ($7a),y	; read character from keyboard buffer

beq $f867	; terminator found

cmp #$22	; quotes?

beq $f872	; yes

cmp #$3a	; colon?

bne $f85a	; nope, next character

.f867  bit $9d	; test MSGFLG, if direct mode

bpl $f875

clc

jsr $aebd

jmp $f878

.f872  jsr $a8fb	; add value in (Y) to TXTPTR

.f875  jsr $ad9e	; evaluate expression in text

.f878  jsr $e25a	; use part of OPEN/CLOSE to SETNAM

jsr $79	; CHRGET

cmp #$2c	; test for comma ','

bne $f887	; nope

jsr $b79b	; use GTBYTC to read character after comma

.f885  stx $ba	; store in FA, device number

.f887  ldy #$00

bit $9d	; test MSGFLG, if direct mode

bpl $f89a

.f88d  lda ($d1),y	; current screen line address, read from screen

cmp #$20	; space

beq $f89a	; yepp

lda #$0d	; carriage return

jsr $e716	; output to screen

bne $f88d

.f89a  jsr $f75c	; test if device FA is present

lda #$ff

jsr $b475

lda $b7	; FNLEN

ldx $bb	; FNADR, pointer to current filename

ldy $bc

jsr $b4c7

jsr $b6a3	; do string housekeeping

stx $bb	; store in FNADR, pointer to current filename

sty $bc

F8B2 OPEN COMMAND CHANNEL

The following routine open the command channel. A test is done to see if it is allready open. If so, the command channel is closed before opened.

.f8b2  jsr $f2cf	; close command channel if open

lda $b7	; read FNLEN, length of current filename, temp store

ldx #$00	; store 0

stx $b7	; in FNLEN

ldx #$6f

bne $f8c3	; allways jump

ldx #$6e

lda $b7

.f8c3  stx $b9	; store in SA, current secondary address

stx $9f	; store in JiffyDOS default filenamber

.f8c7  pha

stx $b8	; store in LA, current logical file number

jsr $ffcc	; CLRCHN, close all I/O channels

jsr $ffc0	; OPEN

pla

sta $b7	; restore FNLEN, length of current filename

.f8d3  rts	; return

F8D4 JIFFYDOS LOCK/UNLOCK FILE

This routine locks/unlocks specifyed file. The file is opened, and tests are made to check that everything is OK. If so a bunch of code are transfered to the drive, and executed. The code to be transfered is found at $f398, after the memory-write command.

.f8d4  jsr $f1e8	; open file and test if all is OK

bne $f8d3	; not ok

ldx #$00	; setup drivecommand at $f398+0.

ldy #$22	; length of string

jsr $f8e4	; execute

ldy #$05	; setup drivecommand at $f398+$22, length 5 bytes

ldx #$22

.f8e4  jsr $f0f4	; execute direct drivecommand

jmp $ffcc	; CLRCHN, close all I/O channels

F8EA JIFFYDOS PATCH, SERIAL SEND

This is a patch to the original Commodore KERNAL to send data on the serial bus.

.f8ea  sta $dd00	; store in serial bus I/O port

and #$08	; test ATN, attension

beq $f910	; ATN = 1

lda $95	; BSOUR

ror a

ror a

cpx #$02	; bit counter =2

bne $f910	; if not, exit

ldx #$1e

.f8fb  bit $dd00

bpl $f905

dex

bne $f8fb

beq $f90e

.f905  bit $dd00

bpl $f905

ora #$40

sta $a3

.f90e  ldx #$02

.f910  rts

F911 JIFFYDOS DISPLAY ASCII FILE

The following routine is called by the LIST ASCII from disk. It clears the command channel and calls a routine that reads maximum 254 character from  the file. This is repeated until the entire file is displayed.

.f911  ldy #$00

jsr $f0e2	; CLRCHN and perform CHKIN on (A)

.f916  jsr $fca9	; read text into buffer

bvs $f91d	; finish

bcc $f916	; next

.f91d  sty $b7	; FNLEN, length of current filename

lda $90	; STATUS

and #$82

rts

F924 JIFFYDOS INTERLEAVE

The following routine sets the interleave gapsize  by writing the selected value to drive memory at possition $0069.

.f924  jsr $b79b	; GEBYTC, getbyte from keyboard buffer

txa	; transfer gapsize to (A)

ldx #$2d	; setup drive command at $f398+2d, M-W 69 00 01

bne $f930	; jump always

F92C JIFFYDOS BUMP DISABLE

The following routine disables the 1541 head rattle. This is done by writing the value $85 to drivememory at position $006a.

.f92c  lda #$85

ldx #$27	; setup drive command at $f398+$27, M-W 6a 00 01

.f930  ldy #$06

pha

jsr $f0f4	; execute drive command

pla

jmp $ffd2	; write byte in (A) to drive, and return

F93A JIFFYDOS MARK FILE FOR COPY

This routine toggles the copy flag for one file, of for all selected files depending on the entry point. If entry at $f93a, the copy flags for all files will be toggled, and if entry at $f93d only one will be affected.

.f93a  ldx #$00	; toggle flag for all files

.byte $2c	; mask LDX-command

.f93d  ldx #$06	; toggle flag for current file

jsr $a68e	; STXPT, reset TXTPTR to start of BASIC

ldy #$05

lda ($7a),y	; test 5:th character

cmp #$12	; <RVS ON>?

bne $f9b0	; if not, directory not loadad, exit

pla

txa	; store (X), the toggle flag, on stack

pha

ldy #$23	; set offset to $23

.f94f  ldx #$22	; search for a quote marks (")

jsr $a917	; use part of DATAN, to search for character

dey

jsr $a8fb	; add offset in (Y) to TXTPTR

pla	; read flag, set at start

pha

beq $f96c	; 'toggle all files' are set

sta $d3

ldy #$01

.f960  iny

jsr $f16a	; use part of 'input from screen'

cmp ($7a),y

bne $f977

sbc #$22

bne $f960

.f96c  tay

lda ($7a),y	; get character

eor #$0a	; toggle between $20 (space) and $2a (*)

sta ($7a),y	; store character

ldy #$04

sta ($d1),y

.f977  jsr $a8f8	; DATA, perform data, skip line like REM

ldy #$05

sec

lda ($7a),y

sbc #$42

bne $f94f	; next line

ldy #$02

sta ($7a),y

pla	; set flag, read from stack

beq $f98d	; if zero, all files were marked/unmarked, do LIST

lda #$8d

rts

.f98d  jmp $a6a4	; perform LIST

F990 JIFFYDOS TOGGLE DRIVE COMMANDS

This routine is continued from JiffyDOS get character. It tests if the keys <CTRL D> are pressed. If so, it increments the internal device counter and tests if it is present. The routine will return the new device number in (X), which will be printed, and the routine exits. If <CTRL D> were not pressed, it continues to test <CTRL A> and <CTRL W>. If not, the routine continues to the funktion key test.

.f990  bit $9d	; test MSGFLG

bpl $f9b0	; exit

tsx

ldy $0107,x

cpy #$e1

bne $f9b0	; exit

cmp #$04	; test code #$04, <CTRL D>, toggle drive

bne $f9b2	; if not, jump to next test

inc $be	; increment JiffyDOS device counter

jsr $f73d	; test device number in $be, output (X)

lda #$00

jsr $bdcd	; print numeric value in (A/X)

jsr $aad7	; output CR/LF

jsr $f79a	;

.f9b0  pla	; retrieve (A)

rts	; and exit

.f9b2  cmp #$01	; test code #$01, <CTRL A>, toggle all files for copy

beq $f93a	; toggle all files copy

cmp #$17	; test code #$01, <CTRL W>, toggle one file for copy

beq $f93d	; toggle single file copy

F9BA JIFFY DOS FUNKTION KEYS

This routine test if a shifted, or unshifted funktion key were pressed. If so, it sends a string containing the command to the keyboard buffer. The vector in $b0 points to the command sting table. The strings are in numerical order, and seperated by a null byte. To find the right string, the routine counts through them all till it reaches the X:th string.

.f9ba  ldy $9b	; must be zero. Some internal JiffyDOS flag

bne $f9b0	; exit

cmp #$8d	; test keys F1 to F8

bcs $f9b0	; larger than F8, exit

cmp #$85

bcc $f9b0	; less than F1, exit

pla

sbc #$85	; substract #$85

tax	; transfer key number to (X)

beq $f9d5	; if F1, do right away

.f9cc  iny	; increment pointer

lda ($b0),y	; read and skip string in function key table

bne $f9cc	; repeat till last byte in string

dex	; next string

bne $f9cc	; till (X) strings are skipped

.f9d4  iny

.f9d5  lda ($b0),y	; read command from corresponding key

beq $f9e2	; if final character, exit

cmp #$0d	; <return>

beq $f9e4	;

jsr $e716	; output to screen

bne $f9d4	; next character

.f9e2  sta $d4

.f9e4  rts

F9E5 JIFFYDOS GET CHARACTER

This routine is a new JiffyDOS routine to handle extended functions. It is called from $e5ec, and starts with the original jump. The routine test the F-keys, and if a valid combination of <CTRL xx> is pressed. If quote mode or insert mode is activated, then this routine will exit.

.f9e5  jsr $e5b4	; get character from keyboard buffer

pha	; temp store

ldx $d4	; test QTSV, if quote mode is activated

bne $fa37	; if not zero, quote mode is on - exit

ldx $d8	; test INSRT, inseret mode

bne $fa37	; if not zero, insert mode is on - exit

cmp #$10	; test code #$10, <CTRL P>, screen dump

bne $f990	; if not pressed, jump and test other keys

F9F5 JIFFYDOS SCREEN DUMP

This routine performs a screen dump when the keys <CTRL P> are pressed. It reads $d018 to determine if upper or lower character set is used, and sends the proper SA after LISTEN. The routine stores the cursor positions on the stack, and retrieves them, and replaces the cursor on exit. To print a character to the serial bus, the routine uses part of the KERNAL CIOUT routine.

.f9f5  lda #$04	; printer device #4

jsr $ffb1	; send LISTEN to device #4

lda $d018	; test upper/lower character set

and #$02

beq $fa03

lda #$07	; set SA=#$67

.fa03  ora #$60	; set SA=#$60

jsr $ff93	; send SA after LISTEN

lda $d3	; PNTR, cursor column

pha	; temp store

lda $d6	; TBLX, cursor line

pha	; temp store

.fa0e  ldy #$00	; column counter

sty $d4	; clear quotes mode, by writing zero into QTSW

jsr $e50c	; PLOT, put row and column

inc $d5	; increment LNMX, maximum screen line length

.fa17  jsr $f16a	; input from screen

jsr $eddd	; CIOUT, send data to serial bus (printer)

cmp #$0d	; carridge return

bne $fa17	; next character

inx	; increment (X), line number

cpx #$19	; till all 25 are done

bcs $fa2d	; exit

asl $d5

bpl $fa0e	; next line

inx

bne $fa0e	; next line

.fa2d  jsr $ffae	; UNLISTEN

pla	; retrieve (X) and (Y)

tax

pla

tay

jsr $e50c	; PLOT, put cursor on same position as on entry

.fa37  pla	; return to original 'get character' routine with key

.fa38  rts	; code in (A)

FA39 JIFFYDOS COPY COMMAND

The following routine is executed to copy files.

.fa39  sty $26	; (Y) =0, temp store

jsr $f1e8	; open command channel and read status

bne $fa38	; not OK, exit

jsr $79	; CHARGET

cmp #$52	; R

bne $fa5a

.fa47  dec $26

lda $26

jsr $f66b

jsr $e4c6	; input byte from command channel, and compare to 5

beq $fa47	; yepp

lda #$00

jsr $f66b

lda #$4c	; L

.fa5a  pha

ldx $bf

cpx $ba	; compare to FA, current device number

beq $fa37	; exit

jsr $f885

ldx #$37	; setup drive command at $f398+$37

ldy #$02	; 2 bytes long

jsr $f0f4	; send S:

jsr $f5c1	; print filename

lda #$2c	; ,

sta ($bb),y	; store in filename buffer

iny

pla	; retrieve command

sta ($bb),y	; store in filename buffer

iny

lda #$2c

sta ($bb),y	; ,

iny

lda $26

pha

bne $fa83

lda #$57

.fa83  sta ($bb),y	; W

iny

sty $b7	; update FNLEN, legnth of current filename

ldy #$0c

.fa8a  jsr $fab2	; set SA to (Y) and more

jsr $f73d	; test for present device

jsr $f8b2	; open command channel

pla

jsr $f541	; use list ASCII from disk to perform copy

FA97 TOGGLE PRINTER

The following routine toggles the printer output funktion. It reads the CHANNL to determine if printmode is to be turned on or off.

.fa97  lda $13	; CHANNL, contains 00 if current output is screen

beq $faa7	; toggle printer on

cmp #$7f	; CHANNL, contains 7f if current output is printer

bne $fa38	; jump to RTS

jsr $abb7	; CLRCHN, clear all channels, and set CHANNL=0

lda #$7f

jmp $ffc3	; close file $7f

.faa7  ldx #$04	; devicenumber #4 = printer

jsr $73	; CHRGET??

jsr $e229	; use part of OPEN routine to open dev#4

jsr $f75c	; test device number in FA

.fab2  sty $b9	; SA, current secondary address

ldx #$7f

stx $13	; CHANNL, current I/O channel

lda $b7	; FNLEN, length of current filename

jmp $f8c7	; perform CLRCHN and OPEN file (X)

tax

bne $fa8a

lda $b5

beq $face

FAC4 PATCH TO ORIGINAL ”LOAD” ROUTINE

This routine is a patch to the original load routine and tests is the current device is a JiffyDOS device. If not, the routine jumps back to the original loader at $f4f3. The routine disables the sprites and calculates the timing parameters to $b1. Some handshaking is done

.fac4  jsr $f5d2	; print ”LOADING/VERIFYING”

tsx	; test if some return pointer on the stack is $f7

lda $0102,x

cmp #$f7

bne $fad7	; if not, don’t store the $ae/$af parameters

lda $ae

sta $55

lda $af

sta $56

.fad7  bit $a3	; ldflg, are we talking to a JiffyDOS device?

bmi $fade	; yes

jmp $f4f3	; nope, return to the original loadroutine.

.fade  sei	; no interrupts

ldy #$03

.fae1  lda $af,y	; save $b0,$b1,$b2 on the stack

pha

dey

bne $fae1

lda $d015	; any sprites enabled?

sta $b0	; store

jsr $f0d8	; clear all sprites not to mess up the timing!

.faf0  jsr $f6bc	; <STOP> key pressed?

bpl $fb27	; yes - exit

lda $d011	; read finscroll

and #$07	; mask bits

clc

adc #$2f	; add $2f - start of the visible screen

sta $b1	; store

lda $dd00	; read and store the lower three bits in $dd00

and #$07	; they contain PA2 and gfxbank pointers

sta $b2

sta $dd00	; clear

ora #$20	; %00100000

tax

.fb0c  bit $dd00	; bit test

bvc $fb0c	; loop if input clk=0

bpl $fb3e	; goto LOADER if input data=0

ldx #$64

.fb15  bit $dd00	; bit test

bvc $fb20	; EOI if input clk=0

dex

bne $fb15	; repeat $64 times

lda #$42	; status code $42, EOI & READ TIMEOUT

.byte $2c

.fb20  lda #$40	; status code $40, EOI

jsr $fe1c	; set STATUS

clc	; clear carry

.byte $24	; mask

.fb27  sec	; set carry

lda $b0	; enable sprites

sta $d015

pla	; restore zero page addresses

sta $b0

pla

sta $b1

pla

sta $b2

bcs $fb3b	; if carry set, exit the normal way

jmp $f528	;

.fb3b  jmp $f633	; exit and unlisten

FB3E THE JIFFYDOS XFER ROUTINE FOR LOAD

.fb3e  bit $dd00	; drive timing

bpl $fb3e	; loop if input data=0

sec

.fb44  lda $d012	; raster timing

sbc $b1

bcc $fb4f

and #$07

beq $fb44

.fb4f  lda $b2	; lower three bits of $dd00

stx $dd00	; store %00100xxx in $dd00

bit $dd00	; bit test

bvc $faf0	; branch is input clk=0

nop	; timing

sta $dd00	; store %00000xxx in $dd00

ora $dd00	; read two first bits

lsr a	; move right two times

lsr a

nop	; timing

ora $dd00	; read next two bits

lsr a	; move right two times

lsr a

eor $b2	; ”trixx” to handle the lower three bits of $dd00

eor $dd00

lsr a	; move right two times

lsr a

eor $b2	; ”trixx” to handle the lower three bits of $dd00

eor $dd00

cpy $93	; load/verify flag

bne $fb83	; branch if verify

sta ($ae),y	; store loaded byte in memory

.fb7a  inc $ae	; next low-byte

bne $fb44	; fetch next byte

inc $af	; next high byte

jmp $fb44	; fetch next byte

.fb83  cmp ($ae),y	; verify byte

beq $fb7a	; equal

sec

lda #$10	; verify error

sta $90	; store in STATUS

bne $fb7a	; continue

lda $c2

sta $ad

lda $c1

sta $ac

rts

FB97 JIFFYDOS DISABLE SPRITES BEFORE ACPTR

This routine disables all the sprites on the screen, and continues the loading procedure. Afterwards the sprites are enabled again.

.fb97  pha	; store the $d015 value on the stack

jsr $f0d8	; disable all the sprites

jsr $fbb4	; continue the loader below

pla	; restore

sta $d015	; and enable sprites again

lda $a4	;

rts

FBA5 JIFFYDOS ACPTR

This is the JiffyDOS ACPTR routine which fetches a byte from the serial bus. Entry point is $fbaa where a test is done by checking $a3 to see if the current device is a JiffyDOS device. Visible sprites are disabled, and raster-timing is done so that no serial access is done when there is a ”bad rasterline”

.fba5  lda #$00	; jump back to the normal load routine

jmp $ee16

.fbaa  sei

bit $a3	; test $a3 to see if the device is a JiffyDOS drive

bvc $fba5	; nope, back to normal load routine

lda $d015	; sprites on screen that can mess up the critical 	; timing

bne $fb97	; yes, clear sprites before loading

.fbb4  lda $dd00	; serial bus

cmp #$40	; test bit 6

bcc $fbb4	; loop

and #$07	; mask lower three bits

pha	; store

.fbbe  lda $d012	; current raster line

sbc $d011	; substract fine scroll register

and #$07	; mask upper bits

cmp #$07

bcs $fbbe	; wait a little longer

pla	; restore

sta $dd00	; write output clk=0 and output data=0

sta $a4

ora #$20	; set bit 5=1

pha	; store on stack

nop	; timing

nop

ora $dd00	; first two bits

lsr a	; rotate right

lsr a

nop	; timing

ora $dd00	; next bits

lsr a	; rotate right

lsr a

eor $a4	; take care of the lower three bits in $dd00

eor $dd00	; next bits

lsr a	; rotate right

lsr a

eor $a4	; take care of the lower three bits in $dd00

eor $dd00	; next bits

sta $a4

pla	; restore from stack

bit $dd00	; bit test

sta $dd00	; store

bvc $fc22

bpl $fc1d

lda #$42

jmp $edb2

FBFE PATCH SEND DATA ON SERIAL LINE

The following routine is used to send a byte to a device on ther serial bus. The routine checks if the device is a JiffyDOS device by reading $a3. If not a JiffyDOS device, the routine jumps back to the original load routine at $ed40.

.fbfe  sei	; disable interrupts

bit $a3	; ldflg

bvc $fc14	; test some more

.fc03  lda $d015	; any sprites enabled

beq $fc27	; nope, continue the send byte routine

pha	; store number of sprites on the stack

jsr $f0d8	; disable all sprites

jsr $fc27	; send-byte routine

pla	; read stack

sta $d015	; enable sprites

rts	; return

.fc14  lda $a3	; ldflg

cmp #$a0

bcs $fc03	; go and test sprites

jmp $ed40	; original send data on serial bus

.fc1d  lda #$40	; %01000000 (EOI)

jsr $fe1c	; set I/O status

.fc22  lda $a4

.fc24  cli

clc

rts

FC27 JIFFYDOS PATCH SEND DATA ON SERIAL LINE

The bits in BSOUR are sent in the following order %22114334.

.fc27  txa	; store (X) on the stack

pha

lda $95	; BSOUR, the byte to send

and #$f0	; upper four bits

pha	; on stack

lda $95	; BSOUR, the byte to send

and #$0f	; lower four bits

tax	; to (X)

.fc33  lda $dd00	; serial bus

bpl $fc33	; loop as long as data input=0

and #$07	; %00000111, mask lower three bits, PA2 and gfx bank

sta $95	; store

sec

.fc3d  lda $d012	; time the send routine with the raster

sbc $d011	; badlines are not allowed durin xfer

and #$07

cmp #$06

bcs $fc3d	; wait

lda $95	; 00000xxx

sta $dd00	; clear serial bus to ”clock” the drive

pla	; upper four bits to send

ora $95	; set PA2 and gfx bank

sta $dd00	; send to drive over serial bus

lsr a	; next two bits

lsr a

and #$f0	; clear low nybble

ora $95	; and set PA2 and gfx bank

sta $dd00	; send to drive over serial bus

lda $fc8a,x	; use (X) as offset for lownybble-table

ora $95	; set PA2 and gfx bank

sta $dd00	; send to drive over serial bus

lsr a	; next two bits

lsr a

and #$f0	; clear low nybbls

ora $95	; set PA2 and gfx bank

sta $dd00	; send to drive over serial bus

and #$0f

bit $a3

bmi $fc76

ora #$10

.fc76  sta $dd00

pla	; restore (X)

tax

lda $95	; PA2 and gfk bank

ora #$10	; set send clk=1

sta $dd00	; store

bit $dd00	; read serial bus

bpl $fc24	; branch if input data=0

jmp $edb0	; back

FCA8 JIFFYDOS SENDTABLE

A table of bit combinations for the lower nybble of the byte to send to a JiffyDOS device.

.fc8a  .byte %00000000	; $00

.byte %10000000	; $80

.byte %00100000	; $20

.byte %10100000	; $a0

.byte %01000000	; $40

.byte %11000000	; $c0

.byte %01100000	; $60

.byte %11100000	; $e0

.byte %00010000	; $10

.byte %10010000	; $90

.byte %00110000	; $30

.byte %10110000	; $b0

.byte %01010000	; $50

.byte %11010000	; $d0

.byte %01110000	; $70

.byte %11110000	; $f0

beq $fcbb

ldx #$f7

jmp $f5c1

FCA0 JIFFYDOS X COMMAND

The following routine sets the  destination devicenumber when using the JiffyDOS copyroutine.

.fca0  jsr $b79b	; GTBYTC, get destination device

stx $bf	; store in $bf

rts	; back

FCA6 READ INTO BUFFER

The following routine is used by the LIST ASCII and LIST BASIC directly from disk. It reads a number of bytes into the filename buffer area.

.fca6  jsr $f0e2	; CLRCHN, and perform CHKIN on (A)

.fca9  jsr $ffcf	; CHRIN

sta ($bb),y	; FNADR POINTER, store in buffer for current filename

iny	; next character

bit $90	; test STATUS

bvs $fcbb	; exit

cpy #$fe	; max length

bcs $fcbb	; yepp

cmp #$01	; larger than 1

bcs $fca9	; yepp, repeat

.fcbb  rts

FCBC DISABLE JIFFYDOS COMMANDS

The following routine is called by the @X command and restores the IERROR, IMAIN and ICRNCH vector.

.fcbc  ldx #$05

.fcbe  lda $f1a3,x	; table with original vectors

sta $0300,x	; store in vector table

dex

.fcc5  bpl $fcbe

stx $9b	; (X)=255, JiffyDOS not activated.

rts

lda aa5

ora (p29,x)

sbc f0185,x

sec

lda aac

sbc aae

lda aad

sbc aaf

rts

inc aac

bne ifce1

inc aad

ifce1  rts

FCE2 POWER RESET ENTRY POINT

The system hardware reset vector ($FFFC) points here. This is the first routine executed when the computer is switched on. The routine firstly sets the stackpointer to #ff, disables interrupts and clears the decimal flag. It jumps to a routine at $fd02 which checks for autostart-cartridges. If so, an indirectjump is performed to the cartridge coldstart vector at $8000. I/O chips are initiated, and system constants are set up. Finaly the IRQ is enabled, and an indirect jump is performed to $a000, the basic cold start vector.

Future implementaions? - A patch to disable the $8000 autostart if a special key is pressed.

.fce2  ldx #$ff

sei

txs	; Set stackpointer to #ff

cld

.fce7  jsr $fd02	; Check ROM at $8000

bne $fcef

jmp ($8000)	; Jump to autostartvector

.fcef  stx $d016

jsr $fda3	; Init I/O

jsr $fd50	; Init system constants

jsr $fd15	; KERNAL reset

jsr $ff5b	; Setup PAL/NTSC

cli

jmp ($a000)	; Basic coldstart

FD02 CHECK FOR 8-ROM

Checks for the ROM autostartparametrar at $8004-$8008. It compares data with $fd10, and if equal, set Z=1.

.fd02  ldx #$05	; 5 bytes to check

.fd04  lda $fd0f,x	; Identifyer at $fd10

cmp $8003,x	; Compare with $8004

bne $fd0f	; NOT equal!

dex

bne $fd04	; until Z=1

.fd0f  rts

FD10 8-ROM IDENTIFYER

The following 5 bytes contains the 8-ROM identifyer, reading "CBM80" with CBM ASCII. It is used with autostartcartridges. See $fd02.

.fd10  c3 c2 cd 38 30	; CBM80

FD15 RESTOR: KERNAL RESET

The KERNAL routine RESTOR ($ff8a) jumps to this routine. It restores (copys) the KERNAL vectors at $fd30 to $0314-$0333. Continues through VECTOR.

.fd15  ldx #$30

ldy #$fd	; $fd30 - table of KERNAL vectors

clc	; Clear carry to SET values.

FD1A VECTOR: KERNAL MOVE

The KERNAL routine VECTOR ($ff8d) jumps to this routine. It reads or sets the vactors at $0314-$0333 depending on state of carry. X/Y contains the adress to read/write area, normally $fd30. See $fd15.

A problem is that the RAM under the ROM at $fd30 always gets a copy of the contents in the ROM then you perform the copy.

.fd1a  stx $c3	; MEMUSS - c3/c4 temporary used for adress

sty $c4

ldy #$1f	; Number of bytes to transfer

.fd20  lda $0314,y

bcs $fd27	; Read or Write the vectors

lda ($c3),y

.fd27  sta ($c3),y

sta $0314,y

dey

bpl $fd20	; Again...

rts

FD30 KERNAL RESET VECTORS

These are the vectors that is copyed to $0314-$0333 when RESTOR is called. These vectors are the same in JiffyDOS, as in stock Commodore KERNAL.

.fd30  31 ea	; CINV VECTOR: hardware interrupt ($ea31)

.fd32  66 fe	; CBINV VECTOR: software interrupt ($fe66)

.fd34  47 fe	; NMINV VECTOR: hardware nmi interrupt ($fe47)

.fd36  4a f3	; IOPEN VECTOR: KERNAL open routine ($f3a4)

.fd38  91 f2	; ICLOSE VECTOR: KERNAL close routine ($f291)

.fd3a  0e f2	; ICHKIN VECTOR: KERNAL chkin routine ($f20e)

.fd3c  50 f2	; ICKOUT VECTOR: KERNAL chkout routine ($f250)

.fd3e  33 f3	; ICLRCH VECTOR: KERNAL clrchn routine ($f333)

.fd40  57 f1	; IBASIN VECTOR: KERNAL chrin routine ($f157)

.fd42  ca f1	; IBSOUT VECTOR: KERNAL chrout routine ($f1ca)

.fd44  ed f6	; ISTOP VECTOR: KERNAL stop routine ($f6ed)

.fd46  3e f1	; IGETIN VECTOR: KERNAL getin routine ($f13e)

.fd48  2f f3	; ICLALL VECTOR: KERNAL clall routine ($f32f)

.fd4a  66 fe	; USRCMD VECTOR: user defined ($fe66)

.fd4c  a5 f4	; ILOAD VECTOR: KERNAL load routine ($f4a5)

.fd4e  ed f5	; ISAVE VECTOR: KERNAL save routine ($f5ed)

FD50 RAMTAS: INIT SYSTEM CONSTANTS

The KERNAL routine RAMTAS($ff87) jumps to this routine. It clears the pages 0,2 and 3 by writing 00 into them. It also sets the start of the cassette buffer - $033c, and determins how much free RAM-memory there is. (The tapebuffer could probably be removed, since JiffyDOS doesn't use tapes at all.) The memorycheck is performed by writing two different bytes into all memory positions, starting at $0400, till it reaches the ROM (the byte read is not the same as the one you wrote.) Note that the contents of the memory is restored afterwards. Finally, bottom of the memory, and top of screen-pointers are set.

Future implementations? - Make a faster RAMcheck routine which not reads all bytes from $0400 and upwards. There can only be ROM at $8000 to $a000, so why bother to check elsewhere. Save a few bytes ad lots of time!!

.fd50  lda #$00

tay

.fd53  sta $02,y	; Fill pages 0,2,3 with zeros

sta $0200,y

sta $0300,y

iny

bne $fd53	; all 256 bytes

ldx #$3c

ldy #$03	; Set tapebuffer to $033c

stx $b2	; Variables TAPE1 is used.

sty $b3

tay

lda #$03

sta $c2

.fd6c  inc $c2

.fd6e  lda ($c1),y	; Perform memorytest. Starting at $0400 and upwards.

tax	; Store temporary in X-reg

lda #$55

sta ($c1),y	; Write #$55 into memory

cmp ($c1),y	; and compare.

bne $fd88	; if not equal... ROM

rol a

sta ($c1),y	; Write #$AA into same memory

cmp ($c1),y	; and compare again.

bne $fd88	; if not equal... ROM

txa

sta ($c1),y	; Restore stored value

iny

bne $fd6e	; Next memorypos

beq $fd6c	; New page in memory

.fd88  tya	; The memorytest always exits when reaching a ROM

tax

ldy $c2

clc

jsr $fe2d	; Set top of memory. X and Y holds address.

lda #$08

sta $0282	; Set pointer to bottom of memory ($0800)

lda #$04

sta $0288	; Set pointer to bottom of screen ($0400)

rts

FD96 TAPE IRQ VECTORS

This table contains the vectors to the four tape-IRQ routines. The vectors are:  $fc6a - tape write I, $fcbd - tape write II, $ea31 - normal IRQ, $f92c - tape read. This table could probably be removed, to gain another 8 bytes of free ROM for own code.

.fd96  6a fc bd fc 31 ea 2c f9

FDA3 IOINIT: INIT I/O

The KERNAL routine IOINIT ($ff84) jumps to this routine. It sets the init-values for the CIAs (IRQ, DDRA, DRA etc.), the SID-volume, and the processor onboard I/O port.

.fda3  lda #$7f

sta $dc0d	; CIA#1 IRQ control register

sta $dd0d	; CIA#2 IRQ control register

sta $dc00	; CIA#1 data port $ (keyboard)

lda #$08

sta $dc0e	; CIA#1 control register timer A

sta $dd0e	; CIA#2 control register timer A

sta $dc0f	; CIA#1 control register timer B

sta $dd0f	; CIA#2 control register timer B

ldx #$00

stx $dc03	; CIA#1 DDRB. Port B is input

stx $dd03	; CIA#2 DDRB. Port B is input

stx $d418	; No sound from SID

dex

stx $dc02	; CIA#1 DDRA. Port A is output

lda #$07	; %00000111

sta $dd00	; CIA#2 dataport A. Set Videobank to $0000-$3fff

lda #$3f	; %00111111

sta $dd02	; CIA#2 DDRA. Serial bus and videobank

lda #$e7	; 6510 I/O port - %XX100111

sta $01

lda #$2f	; 6510 I/O DDR - %00101111

sta $00

FDDD ENABLE TIMER

This routine inits and starts the CIA#1 timer A according to the PAL/NTSC flag. Different system clocks rates are used in PAL/NTSC systems.

.fddd  lda $02a6	; PAL/NTSC flag

beq $fdec	; NTSC setup

lda #$25

sta $dc04	; CIA#1 timer A - lowbyte

lda #$40	; PAL-setup #4025

jmp $fdf3

.fdec  lda #$95

sta $dc04	; CIA#1 timer A - lowbyte

lda #$42	; NTSC-setup #4295

.fdf3  sta $dc05	; CIA#1 timer A - highbyte

jmp $ff6e	; start timer

FDF9 SETNAM: SAVE FILENAME DATA

The KERNAL routine SETNAM ($ffbd) jumps to this routine. On entry, A-reg holds the length of the filename, and X/Y the address in mem to the filename.

.fdf9  sta $b7	; store length of filename in FNLEN

stx $bb	; store pointer to filename in FNADDR

sty $bc

rts

FE00 SETLFS: SAVE FILE DETAILS

The KERNAL routine SETLFS ($ffba) jumps to this routine. On entry A-reg holds the logical filenumber, X the device number, and Y the secondary address.

.fe00  sta $b8	; store logical filenumber in LA

stx $ba	; store devicenumber in FA

sty $b9	; store secondary address in SA

rts

FE07 READST: READ STATUS

The KERNAL routine READST ($ffb7) jumps to this routine. The routine checks if the current devicenumber is 2, (ie RS232) then the value of RSSTAT (the ACIA 6551 status)is returned in (A), and RSSTAT is cleared. Else it reads and returnes the value of STATUS.

.fe07  lda $ba	; read current device number from FA

cmp #$02	; device = RS232?

bne $fe1a	; nope, read STATUS

lda $0297	; RSSTAT

pha	; temp store

lda #$00

sta $0297	; clear RSSTAT

pla

rts

FE18 SETMSG: FLAG STATUS

The KERNAL routine SETMSG ($ff90) jumps to this routine. On entry, the value in (A) is stored in MSGFLG, then the I/O status is placed in (A). If routine is entered at $fe1c the contents in (A) will be stored in STATUS.

.fe18  sta $9d	; store MSGFLG

.fe1a  lda $90	; read STATUS

.fe1c  ora $90

sta $90

rts

FE21 SETTMO: SET TIMEOUT

The KERNAL routine SETTMO ($ffa2) jumps to this routine. On entry the value in (A) is stored in the IEEE timeout flag. (Who uses IEEE nowadays?)

.fe21  sta $0285	; store in TIMOUT

rts

FE25 MEMTOP: READ/SET TOP OF MEMORY

The KERNAL routine MEMTOP ($ffa9) jumps to this routine. If carry is set on entry, the top of memory address will be loaded into (X/Y). If carry is clear on entry, the top of memory will be set according to the contents in (X/Y)

.fe25  bcc $fe2d	; carry clear?

ldx $0283	; read memtop from MEMSIZ

ldy $0284

.fe2d  stx $0283	; store memtop in MEMSIZ

sty $0284

rts

FE34 MEMBOT: READ/SET BOTTOM OF MEMORY

The KERNAL routine MEMBOT ($ff9c) jumps to this routine. If carry is set on entry, the bottom of memory address will be loaded into (X/Y). If carry is clear on entry, the bottom of memory will set according to the contents in (X/Y)

.fe34  bcc $fe3c	; carry clear?

ldx $0281	; read membot from MEMSTR

ldy $0282

.fe3c  stx $0281	; store membot in MEMSTR

sty $0282

rts

FE43 NMI ENTRY POINT

The processor jumps to this routine every time a NMI occurs (see jump vector at $fffa). On entry all processor registers will be put on the stack. The routine will check the presents of a ROM cartridge at $8000 with autostart, and warm start it. Otherwise, the following warm start routine is called.

.fe43  sei	; disable interrupts

jmp ($0318)	; jump to NMINV, points normally to $fe47

.fe47  pha	; store (A), (X), (Y) on the stack

txa

pha

tya

pha

lda #$7f	; CIA#2 interrupt control register

sta $dd0d

ldy $dd0d

bmi $fe72	; NMI caused by RS232? If so - jump

jsr $fd02	; check for autostart at $8000

bne $fe5e

jmp ($8002)	; Jump to warm start vector

.fe5e  jsr $f6bc	; Scan one row in the keymatrix and store value in $91

jsr $ffe1	; Check $91 to see if <STOP> was pressed

bne $fe72	; <STOP> not pressed, skip part of following routine

FE66 WARM START BASIC

This routine is called from the NMI routine above. If <STOP> was pressed, then KERNAL vectors are restored to default values, I/O vectors initialised and a jump to ($a002), the Basic warm start vector.

The NMI routine continues at $fe72 by checking the RS232, if there is anyting to send.

.fe66  jsr $fd15	; KERNAL reset

jsr $fda3	; init I/O

jsr ee518	; init I/O

jmp ($a002)	; jump to Basic warm start vector

.fe72  tya	; Read CIA#2 interrupt control register

and $02a1	; mask with ENABL, RS232 enable

tax	; temp store in (X)

and #$01	; test if sending (%00000001)

beq $fea3	; nope, jump to recieve test

lda $dd00	; load CIA#1 DRA

and #$fb	; mask bit2 (RS232 send)

ora $b5	; NXTBIT, next bit to send

sta $dd00	; and write to port

lda $02a1

sta $dd0d	; write ENABL to CIA#2 I.C.R

txa	; get temp

and #$12	; test if recieving (bit1), or waiting for reciever 	  edge (bit4) ($12 = %00010010)

beq $fe9d	; nope, skip reciever routine

and #$02	; test if recieving

beq $fe9a	; nope

jsr $fed6	; jump to NMI RS232 in

jmp $fe9d

.fe9a  jsr $ff07	; jump to NMI RS232 out

.fe9d  jsr $eebb	; RS232 send byte

jmp $feb6	; goto exit

.fea3  txa	; get temp

and #$02	; test bit1

beq $feae	; nope

jsr $fed6	; NMI RS232 in???

jmp $feb6	; goto exit

.feae  txa	; set temp

and #$10	; test bit4

beq $feb6	; nope, exit

jsr $ff07	; NMI RS232 out

.feb6  lda $02a1	; ENABL

sta $dd0d	; CIA#2 interrupt control register

pla	; restore registers (Y),(X),(A)

tay

pla

tax

pla

rti	; back from NMI

FEC2 RS232 TIMING TABLE - NTSC

Timingtable for RS232 NMI for use with NTSC machines. The table containe 10 entries which corresponds to one of the fixed RS232 rates, starting with lowest (50 baud) and finishing with the highest (2400 baud). Since the clock frequency is different between NTSC and PAL systems, there is another table for PAL machines at $e4ec.

Future implementations? Remove the table if you run a PAL machine, and put some own code here.

cmp ($27,x)

rol fc51a,x

ora ($74),y

$sl $0ced

eor $06

beq $fed2

lsr $01

.fed2  clv

brk

$dc ($00),y

FED6 NMI RS232 IN

This routine inputs a bit from the RS232 port and sets the baudrate timing for the next bit. Continues to the RS232 recieve routine.

.fed6  lda $dd01	; RS232 I/O port

and #$01	; test bit0, received data

sta $a7	; store in INBIT

lda $dd06	; lowbyte of timer B

sbc #$1c

adc $0299	; <BAUDOF

sta $dd06	; store timer B

lda $dd07	; highbyte of timer B

adc $029a	; >BAUDOF

sta $dd07	; store timer B

lda #$11

sta $dd0f	; CIA#2 control register B

lda $02a1	; ENABL

sta $dd0d	; CIA#2 interrupt control register

lda #$ff

sta $dd06

sta $dd07

jmp $ef59	; jump to RS232 receive routine

FF07 NMI RS232 OUT

This routine sets up the baudrate for sending the bits out, and adjusts the number of bits remaining to send.

.ff07  lda $0295	; M51AJB - non standard BPS time

sta $dd06	; timer B low

lda $0296

sta $dd07	; timer B high

lda #$11

sta $dd0f	; CIA#2 control register B

lda #$12

eor $02a1

sta $02a1	; ENABL, RS232 enables

lda #$ff

sta $dd06

sta $dd07	; timer B

ldx $0298	; BITNUM, number of bits still to send in this byte

stx $a8	; BITC1, RS232 bitcount

rts

.ff2f  tax

lda $0296

rol

tay

txa

adc #$c8

sta $0299

tya

adc #$00

sta $029a

rts

FF41 FAKE IRQ

Fake IRQ entry that clears bit4 which is later tested for HW or SW interrupt. This entry will always create a hardware interrupt.

nop	; don't ask me??

nop

php	; store processor reg.

pla	; get reg

and #$ef	; clear bit4

pha	; store reg

FF48 IRQ ENTRY

This routine is pointed to by the hardware IRQ vector at $fffe. This routine is able to distinguish between s hardware IRQ, and a software BRK. The two types of interrupts are processed by its own routine.

.ff48  pha	; Store Acc

txa

pha	; Store X-reg

tya

pha	; Store Y-reg

tsx

lda $0104,x	; Read byte on stack written by processor?

and #$10	; check bit 4 to determine HW or SW interrupt

beq $ff58

jmp ($0316)	; jump to CBINV. Points to FE66, basic warm start

.ff58  jmp ($0314)	; jump to CINV. Points to EA31, main IRQ entry point

FF5B CINT: INIT SCREEN EDITOR

The KERNAL routine CINT ($FF81) jumps to this routine. It sets up VIC for operation. The original CINT is at $e518, and this patch checks out if this is a PAL or NTSC machine. This is done by setting the raster compare register to 311, which is the number of scanlines in a PAL machine. If no interrupt occurs, then it's a NTSC machine.

.ff5b  jsr $e518	; original I/O init

.ff5e  lda $d012	; wait for top of screen

bne $ff5e	; at line zero

lda $d019	; Check IRQ flag register if interrupt occured

and #$01	; only first bit

sta $02a6	; store in PAL/NTSC flag

jmp $fddd	; jump to ENABLE TIMER

FF6E START TIMER

This routine starts the CIA#1 timer and jumps into a routine that handles the serial clock.

.ff6e  lda #$81	; Enable IRQ when timer B reaches zero

sta $dc0d	; CIA#1 interrupt controll register

lda $dc0e	; CIA#1 control register A

and #$80

ora #$11	; Force load of timer A values -bit4, and start -bit0

sta $dc0e	; Action!

jmp $ee8e	; Continue to 'serial clock off'

FF80 KERNAL VERSION ID

This byte contains the version number of the KERNAL.

.ff80  sed $

FF81 KERNAL JUMP TABLE

This table contains jump vectors to the I/O routines. This is a Commodore standard, so no matter what system you are using (VIC20, C64, C128, Plus4 etc) the jump vectors are always located at this position.

.ff81  jmp $ff5b	; CINT, init screen editor

.ff84  jmp $fda3	; IOINT, init input/output

.ff87  jmp $fd50	; RAMTAS, init RAM, tape screen

.ff8a  jmp $fd15	; RESTOR, restore default I/O vector

.ff8d  jmp $fd1a	; VECTOR, read/set I/O vector

.ff90  jmp $fe18	; SETMSG, control KERNAL messages

.ff93  jmp $edb9	; SECOND, send SA after LISTEN

.ff96  jmp $edc7	; TKSA, send SA after TALK

.ff99  jmp $fe25	; MEMTOP, read/set top of memory

.ff9c  jmp $fe34	; MEMBOT, read/set bottom of memory

.ff9f  jmp $ea87	; SCNKEY, scan keyboard

.ffa2  jmp $fe21	; SETTMO, set IEEE timeout

.ffa5  jmp $fbaa	; ACPTR, input byte from serial bus. JiffyDOS poits  	  to $fbaa, the original to $ee13.

.ffa8  jmp $eddd	; CIOUT, output byte to serial bus

.ffab  jmp $edef	; UNTALK, command serial bus UNTALK

.ffae  jmp $edfe	; UNLSN, command serial bus UNLSN

.ffb1  jmp $ed0c	; LISTEN, command serial bus LISTEN

.ffb4  jmp $ed09	; TALK, command serial bus TALK

.ffb7  jmp $fe07	; READST, read I/O status word

.ffba  jmp $fe00	; SETLFS, set logical file parameters

.ffbd  jmp $fdf9	; SETNAM, set filename

.ffc0  jmp ($031a)	; OPEN, open file

.ffc3  jmp ($031c)	; CLOSE, close file

.ffc6  jmp ($031e)	; CHKIN, prepare channel for input

.ffc9  jmp ($0320)	; CHKOUT, prepare channel for output

.ffcb  jmp ($0322)	; CLRCHN, close all I/O

.ffcf  jmp ($0324)	; CHRIN, inpup byte from channel

.ffd2  jmp ($0326)	; CHROUT, output byte to channel

.ffd5  jmp $f49e	; LOAD, load from serial device

.ffd8  jmp $f5dd	; SAVE, save to serial device

.ffdb  jmp $f6e4	; SETTIM, set realtime clock

.ffde  jmp $f6dd	; RDTIM, read realtime clock

.ffe1  jmp ($0328)	; STOP, check <STOP> key

.ffe4  jmp ($032a)	; GETIN, get input from keyboard

.ffe7  jmp ($032c)	; CLALL, close all files and channels

.ffea  jmp $f69b	; UDTIM, increment realtime clock

.ffed  jmp $e505	; SCREEN, return screen organisation

.fff0  jmp $e50a	; PLOT, read/set cursor X/Y position

.fff3  jmp $e500	; IOBASE, return IOBASE address

FFF4 SYSTEM HARDWARE VECTORS

This table contains jumpvectors for system reset, IRQ, and NMI. The IRQ and NMI vectors points to addresses which contains an indirect jump to RAM, to provide user defined routines.

.ffe4

.fffa 43 fe	; NMI hardware vector

.fffc e2 fc	; System reset vector

.fffe 48 ff	; IRQ hardware vector